INITIAL STUDY/

ENVIRONMENTAL CHECKLIST FORM

CITY OF MORENO VALLEY

- Project Title:				Legacy Park Project

PEN16-0092 (PA16-0018) – General Plan Amendment

PEN16-0093 (PA16-0019) – Zone Change

PEN16-0094 (PA14-0052) – Conditional Use Permit

PEN16-0095 (PA14-0053) – Tentative Tract Map 36760

PEN16-0096 (P16-030) – Expanded Initial Study

- Lead Agency Name and Address:	City of Moreno Valley

14177 Frederick Street

Moreno Valley, CA  92553

- Contact Person and Phone Number:	Jeff Bradshaw, Associate Planner

(951) 413-3224

- Project Location:				Southeast corner of Gentian Avenue and Indian Street

- Project Sponsor’s Name and Address:	Mission Pacific Land Company

4100 Newport Beach Place, Ste. #480

Newport Beach, CA  92660

- Existing General Plan Designation:	Residential 5 (37.18 acres) and Residential 30 (15.06 acres)

- Proposed General Plan Designation:	Residential 5 (52.24 acres)

- Existing Zoning:				R5 (37.18 acres) and R30 (15.06 acres)

- Proposed Zoning:				R5 (52.24 acres)

- Description of the Project:

The project proposes to develop the Legacy Park planned community on an approximately 53 acre site.  Applications include a General Plan Amendment from Residential 30 to Residential 5 and a Zone Change from R30 to R5 for a 15.06 acre portion of a 53 acre site.  This project includes an application for Tentative Tract Map 36760 to subdivide the 53 acre site into a total of 221 single family residential lots and a Conditional Use Permit for a Planned Unit Development (PUD).  The PUD application will establish minimum lot sizes of 4,000 and 5,000 square feet and establish unique development standards for future single family residential construction within the community.  The proposed 221 lots does not exceed the allowable density for the R5 zone.  Common amenities include passive open space, trail segments, decorative treatment in Street L at major intersections, and a median in Street L at Gentian.

Off-site improvements that the project will be responsible for completing include:

- Master Plan Storm Drain system Line M2 and associated utility relocation.  Approximately 3,000 feet of off-site improvements in Santiago Drive, Perris Boulevard, and Iris Avenue;

- Master Plan Storm Drain system Line D1.  Approximately 300 feet of off-site improvements in Indian Avenue;

- Park improvements on a 0.85 acre parcel within Lot A TPM 36606 (open space area on adjacent Walmart center site);

- Trail and passive park improvements within the adjacent California Aqueduct easement which is under the authority of the Department of Water Resources (DWR).  This will satisfy General Plan requirements under the City’s Master Plan of Trails for development of the City’s portion of the Juan Bautista De Anza trail system;

- A new traffic signal at the intersection of Perris Boulevard and Santiago Drive;

- Street Improvements outside the map boundary on fronting streets:

o   Gentian Ave. – Raised median;

o   Indian Ave. – Street widening, curb/gutter, and parkway improvements; and

o   Santiago Drive - Street widening, curb/gutter, and parkway improvements.

This project is also conditioned to construct and then convey to the City a public park of approximately two acres in size with amenities that would include play equipment, a picnic shelter, a gazebo, large group barbeques, concrete picnic tables and benches, concrete waste/recycle containers; drinking fountains, walkway security lighting, decorative concrete walkways, decomposed granite walking path; and tubular steel fencing surrounding the park.

- Surrounding Land Uses and Setting:

The project site is bounded by existing single-family tract homes to the west and northwest in the RS-10 zone with minimum lot sizes of 4,500 square feet.  The property immediately to the north is zoned R5 and has been subdivided with a recorded map, Tract Map 22180.  Further to the north are existing single-family tract homes in the R5 zone.  Southwest of the project site are single-family homes in the R5 zone with March Middle School and Rainbow Elementary School located immediately to the south.  Vacant and developed land (non-conforming single-family residences) in the R30 zone is located to the southeast of the project site.

The California Aqueduct bounds the property along its eastern property line with vacant Community Commercial zoned property to the east.  The site to the east was recently approved for development as a Walmart retail center. Additional commercial existing retail centers are located to the southeast at the intersection of Perris Boulevard and Iris Avenue.

March Air Reserve Base is located approximately three-quarters of a mile to the west. The City Corporate Yard is located approximately 1,400 feet to the east.

Overall, the proposed residential development is compatible with the City’s General Plan and existing land uses.

- Have California Native American tribes traditionally and culturally affiliated with the project area requested consultation pursuant to Public Resources Code section 21080.3.1? If so, has consultation begun?

The City received requests for consultation from the following Native American tribes and consultation has begun:

- Agua Caliente Band of Cahuilla Indians;

- Pechanga Band of Luiseno Indians; and

- Soboba Band of Luiseno Indians.

- Other public agencies whose approval is required:

Riverside County Flood Control and Water Conservation District will require an encroachment permit for connecting to existing storm drain infrastructure located in Perris Boulevard and the State of California Department of Water Resources will require an encroachment permit for work alongside and within the easement for the California Aqueduct.

ENVIRONMENTAL FACTORS POTENTIALLY AFFECTED:

The environmental factors checked below(  ) would be potentially affected by this project, involving at least one impact that is a “Potentially Significant Impact” as indicated by the checklist on the following pages.

|  | Aesthetics |  | Greenhouse Gas Emissions |  | Population/Housing |
| --- | --- | --- | --- | --- | --- |
|  | Agricultural Resources |  | Hazards & Hazardous Materials |  | Public Services |
|  | Air Quality |  | Hydrology/Water Quality |  | Recreation |
|  | Biological Resources |  | Land Use/Planning |  | Transportation/Traffic |
|  | Cultural Resources |  | Mineral Resources |  | Utilities/Service Systems |
|  | Geology/Soils |  | Noise |  | Mandatory Findings of Significance |
|  | Tribal Cultural Resources |  |  |  |  |

DETERMINATION: (To be completed by the Lead Agency)

On the basis of this initial evaluation:

| I find that the proposed project COULD NOT have a significant effect on the environment, and a NEGATIVE DECLARATION will be prepared. |  |
| --- | --- |
| I find that although the proposed project could have a significant effect on the environment, there will not be a significant effect in this case because revisions in the project have been made by or agreed to by the project proponent. A MITIGATED NEGATIVE DECLARATION will be prepared. |  |
| I find that the proposed project MAY have a significant effect on the environment, and an ENVIRONMENTAL IMPACT REPORT is required. |  |
| I find that the proposed project MAY have a “potential significant impact” or “potentially significant unless mitigated” impact on the environment, but at least one effect (1) has been adequately analyzed in an earlier document pursuant to applicable legal standards, and (2) has been addressed by mitigation measures based on the earlier analysis as described on attached sheets. An ENVIRONMENTAL IMPACT REPORT is required, but it must analyze only the effects that remain to be addressed. |  |
| I find that although the proposed project could have a significant effect on the environment, because all potentially significant effects (a) have been analyzed in an earlier EIR or NEGATIVE DECLARATION pursuant to applicable standards and (b) have been avoided or mitigated pursuant to that earlier EIR or NEGATIVE DECLARATION, including revisions or mitigation measures that are imposed upon the proposed project, nothing further is required. |  |

_____                                                                                        December 19, 2016

Signature								Date

Jeff Bradshaw, Associate Planner

Printed Name								For

EVALUATION OF ENVIRONMENTAL IMPACTS

- A brief explanation is required for all answers except “No Impact” answers that are adequately supported by the information sources a lead agency cites in the parentheses following each question.  A “No Impact” answer is adequately supported if the referenced information sources show that the impact simply does not apply to projects like the one involved (e.g. the project falls outside a fault rupture zone).  A “No Impact” answer should be explained where it is based on project-specific factors as well as general standards (e.g. the project will not expose sensitive receptors to pollutants, based on a project-specific screening analysis).

- All answers must take account of the whole action involved, including off-site as well as on-site, cumulative as well as project-level, indirect as well as direct, and construction as well as operational impacts.

- Once the lead agency has determined that a particular physical impact may occur, then the checklist answers must indicate whether the impact is potentially significant, less than significant with mitigation, or less than significant.  “Potentially Significant Impact” is appropriate if there is substantial evidence that an effect may be significant.  If there are one or more “Potentially Significant Impact” entries when the determination is made, an EIR is required.

- “Negative Declaration: Potentially Significant Unless Mitigation Incorporated” applies where the incorporation of mitigation measures has reduced an effect from “Potentially Significant Impact” to a “Less Significant Impact.”  The lead agency must describe the mitigation measures, and briefly explain how they reduce the effect to a less than significant level (mitigation measures from “Earlier Analysis,” as described in (5) below, may be cross-referenced).

- Earlier analysis may be used where, pursuant to the tiering, program EIR, or other CEQA process, an effect has been adequately analyzed in an earlier EIR or negative declaration.  Section 15063 (c) (3) (d).  In this case, a brief discussion should identify the following:

(a)	Earlier Analysis Used. Identify and state where they are available for review.

- Impacts Adequately Addressed.  Identify which effects from the above checklist were within the scope of and adequately analyzed in an earlier document pursuant to applicable legal standards, and state whether such effects were addressed by mitigation measures based on the earlier analysis.

- Mitigation Measures.  For effects that are “Less than Significant with Mitigation Measures Incorporated,” describe the mitigation measures which were incorporated or refined from the earlier document and the extent to which they address site-specific conditions for the project.

- Lead agencies are encouraged to incorporate into the checklist references to information sources for potential impacts (e.g. general plans, zoning ordinances).  Reference to a previously prepared or outside document should, where appropriate, include a reference to the page or pages where the statement is substantiated.

- Supporting Information Sources:  A source list should be attached, and other sources used or individuals contacted should be cited in the discussion.

- This is only a suggested form, and lead agencies are free to use different formats; however, lead agencies should normally address the questions from this checklist that are relevant to a project’s environmental effects in whatever format is selected.

- The analysis of each issue should identify: (a) the significance criteria or threshold used to evaluate each question; and (b) the mitigation measure identified, if any, to reduce the impact to less than significance.

| I. AESTHETICS. Would the project: |  |  |  |  |  |  |  |  |  |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- | --- | --- |
| a) Have a substantial adverse effect on a scenic vista? |  |  |  |  |  |  |  |  |  |  |
| The Moreno Valley General Plan identifies scenic highways, panoramic viewsheds, and photographic viewing locations within the aesthetic resource element. The General Plan identifies no scenic roadways or panoramic viewsheds in the project vicinity. The project site is comprised of level topography with no rock outcroppings. As designed and conditioned, the proposed project will have no effect on a scenic vista. |  |  |  |  |  |  |  |  |  |  |
| b) Substantially damage scenic resources, including, but not limited to trees, rock outcroppings, and historic buildings within a state scenic highway? |  |  |  |  |  |  |  |  |  |  |
| The project property topography is flat. Based upon site visits by staff and review of the General Plan, the subject site does not include scenic resources. There are no rock outcroppings, trees or historic buildings on site. There are no scenic highways in the area. The site has been previously disturbed through weed abatement. As designed and conditioned, the proposed project will not substantially damage scenic resources. |  |  |  |  |  |  |  |  |  |  |
| c) Substantially degrade the existing visual character or quality of the site and its surroundings? |  |  |  |  |  |  |  |  |  |  |
| The Legacy Park project proposes to subdivide the 53 acre site for development of a 221 lot planned community within the R5 zoning district. Development of the site will require installation of public street improvements along Gentian Avenue, Indian Street and Santiago Drive frontages, the installation of a new segment of raised landscaped median and the undergrounding of overhead utility lines. The Legacy Park Design Guidelines and the Municipal Code provide a framework that ensures that any new development would be designed and constructed in a manner that is compatible with surrounding land uses. The Planned Unit Development and related Design Guidelines provide a framework for coordinating architectural style, design, materials, colors, perimeter walls, pedestrian access and circulation for the development. The proposed project as designed is aesthetically compatible with adjacent residential uses. The Planned Unit Development and related Design Guidelines provide a framework for coordinating architectural style, design, materials, colors, perimeter walls, pedestrian access and circulation for the development. As designed and conditioned, the proposed project would not significantly degrade the existing visual character or quality of the site and surroundings. |  |  |  |  |  |  |  |  |  |  |
| d) Create a new source of substantial light or glare which would adversely affect day or nighttime views in the area? |  |  |  |  |  |  |  |  |  |  |
| The project would introduce some additional new light sources into the area as the project site is currently vacant. The proposed residential development would include required street lighting and exterior wall mounted lights on the residences. The project has been conditioned for compliance with the City’s light standards as referenced in Municipal Code Section 9.08.100 including the shielding of lighting and restrictions on the intensity of exterior lighting which will reduce light and glare impacts to City accepted levels on surrounding properties. Therefore, potential impacts related to substantial light or glare are less than significant and no mitigation would be required. |  |  |  |  |  |  |  |  |  |  |
| II. AGRICULTURE & FORESTRY RESOURCES: In determining whether impacts to agricultural resources are significant environmental effects, lead agencies may refer to the California Agricultural Land Evaluation and Site Assessment Model (1997) prepared by the California Dept. of Conservation as an optional model to use in assessing impacts on agriculture and farmland. In determining whether impacts to forest resources, including timberland, are significant environmental effects, lead agencies may refer to information compiled by the California Department of Forestry and Fire Protection regarding the state’s inventory of forest land, including the Forest and Range Assessment Project and the Forest Legacy Assessment project; and forest carbon measurement methodology provided in Forest Protocols adopted by the California Air Resources Board. Would the project? |  |  |  |  |  |  |  |  |  |  |
| a) Convert Prime Farmland, Unique Farmland or Farmland of Statewide Importance (Farmland), as shown on the maps prepared pursuant to the Farmland Mapping and Monitoring Program of the California Resources Agency to non-agricultural use? |  |  |  |  |  |  |  |  |  |  |
| The site is designated as ‘Farmland of Local Importance’ on the 2015 State Important Farmland Map. This category is described as soils that would be classified as Prime and Statewide but lack available irrigation water. The site is bounded on the north and west by existing residential development with an approved retail center to the east and a middle school and elementary school and single-family residences to the south. There are currently no agriculturally productive activities occurring within the project boundaries. There will be no impact to farmlands as the development of this project will not result in the conversion of Prime Farmland, Unique Farmland or Farmland of Statewide Importance. |  |  |  |  |  |  |  |  |  |  |
| b) Conflict with existing zoning for agricultural use, or a Williamson Act contract? |  |  |  |  |  |  |  |  |  |  |
| The site is not currently in agricultural use, or under Williamson Act control. There is no existing surrounding agricultural use, or sites under Williamson Act contract within the City limits. The Municipal Code allows for agricultural uses such as crops in all zoning districts, therefore, the proposed project does not conflict with existing zoning for agricultural use, or impact sites under Williamson Act contract. |  |  |  |  |  |  |  |  |  |  |
| c) Conflict with existing zoning for, or cause rezoning of, forest land (as defined in Public Resources Code section 12220(g)), timberland (as defined by Public Resources Code section 4526), or timberland zoned Timberland Production (as defined by Government Code section 51104(g))? |  |  |  |  |  |  |  |  |  |  |
| The project site is not zoned or designated on the City’s General Plan for forest land, timberland, or timberland zoned Timberland Production. The City does not have any forest lands, or timberland as defined in the State Public Resources Code and Government Code within the City limits. Therefore, since the project will not result in impacts to forest land, timberland, or timberland zoned timberland production, no impacts would occur and no mitigation measures would be required. |  |  |  |  |  |  |  |  |  |  |
| d) Result in the loss of forest land or conversion of forest land to non-forest use? |  |  |  |  |  |  |  |  |  |  |
| The project site is not forest land as defined by Public Resources Code section 1220(g). The project site does not involve the loss of forest land or the conversion of forest land to non-forest use. Therefore, since the project will not result in the loss of forest land or the conversion of forest land to non-forest use, no impacts would occur and no mitigation measures would be required. |  |  |  |  |  |  |  |  |  |  |
| e) Involve other changes in the existing environment which, due to their location or nature, could result in conversion of Farmland, to non-agricultural use or conversion of forest land to non-forest use? |  |  |  |  |  |  |  |  |  |  |
| There is no immediate surrounding or proposed agricultural use. The proposed project will not involve changes to the existing environment, which will result in the conversion of farmland to non-agricultural use, or conversion of forest land to non-forest land. |  |  |  |  |  |  |  |  |  |  |
| III. AIR QUALITY: Where available, the significance criteria established by the applicable air quality management or air pollution control district may be relied upon to make the following determinations. Would the project: |  |  |  |  |  |  |  |  |  |  |
| a) Conflict with or obstruct implementation of the applicable air quality plan? |  |  |  |  |  |  |  |  |  |  |
| b) Violate any air quality standard or contribute substantially to an existing or projected air quality violation. |  |  |  |  |  |  |  |  |  |  |
| (a and b) The Air Quality Management Plan (AQMP) adopted by the South Coast Air Quality Management District (SCAQMD) in 2012 sets forth a comprehensive program that will lead the air basin into compliance with all federal and state air quality standards. The proposed project is located within the boundaries of the AQMP. The AQMP control measures and related emission reduction estimates are based upon emissions projections for a future development scenario derived from General Plan land use, population, and employment characteristics defined in consultation with local governments. Moreno Valley’s General Plan Land Use Element was considered in the preparation of the 2012 AQMP. Accordingly, conformance with the AQMP for development projects is determined by demonstrating compliance with local land use plans and/or population projections. Based upon the conclusions of the Air Quality study prepared for this project by Urban Crossroads on November 3, 2016, the Project would not result in or cause federal and/or state ambient air quality standards (NAAQS or CAAQS) violations. The Project proposed General Plan Amendment and Zone Change from R30 to R5 would not increase the density allowed in the General Plan and therefore not result in a land use that is more intense than that anticipated by the General Plan. Furthermore, the Project would not exceed any applicable regional or local thresholds. As such, the Project is therefore considered to be consistent with the AQMP. Construction-Source Emissions For regional emissions, the Project would not exceed the numerical thresholds of significance established by the SCAQMD for any criteria pollutant. It should be noted that impacts without mitigation take credit for reductions achieved through standard regulatory requirements (Rule 403 and Rule 1113). Thus a less than significant impact would occur for Project-related construction-source emissions and no mitigation measures are required. For localized emissions, the Project would not exceed the SCAQMD’s localized significance threshold. Therefore, a less than significant impact would occur and no mitigation is required. Project construction-source emissions would not conflict with the applicable AQMP. Operation-Source Emissions For regional emissions, the Project would not exceed the numerical thresholds of significance established by the SCAQMD. Thus a less than significant impact would occur for Project-related operational-source emissions and no mitigation is required. Project operational-source emissions would not result in or cause a significant localized air quality impact as discussed in the operational LSTs section of this report. The proposed Project would not result in a significant CO “hotspot” as a result of Project related traffic during ongoing operations, nor would the Project result in a significant adverse health impact as discussed in Section 3.8, thus a less than significant impact to sensitive receptors during operational activity is expected. |  |  |  |  |  |  |  |  |  |  |
| c) Result in a cumulatively considerable net increase of any criteria pollutant for which the project region is non-attainment under an applicable federal or state ambient air quality standard (including releasing emissions which exceed quantitative thresholds for ozone precursors)? |  |  |  |  |  |  |  |  |  |  |
| CEQA Section 21100 (e) addresses evaluation of cumulative effects allowing the use of approved land use documents in a cumulative impact analysis. CEQA Guidelines Section 15064 (i)(3) further stipulates that for an impact involving a resource that is addressed by an approved plan or mitigation program, the lead agency may determine that a project’s incremental contribution is not cumulatively considerable if the project complies with the adopted plan or program. In addressing cumulative effects for air quality, the AQMP is the most appropriate document to use because the AQMP sets forth a comprehensive program that will lead the air basin, including the project area, into compliance with all federal and state air quality standards and utilizes control measures and related emission reduction estimates based upon emissions projections for a future development scenario derived from land use, population, and employment characteristics defined in consultation with local governments. The Air Quality Management Plan (AQMP) sets forth a comprehensive program that will lead the air basin into compliance with all federal and state air quality standards. The AQMP control measures and related emission reduction estimates are based upon emissions projections for a future development scenario derived from General Plan land use, population, and employment characteristics defined in consultation with local governments. Accordingly, conformance with the AQMP for development projects is determined by demonstrating compliance with local land use plans and/or population projections. The AQMD uses the same significance thresholds for project specific and cumulative impacts for all environmental topics analyzed in an Environmental Assessment or EIR. Projects that exceed the project-specific significance thresholds are considered by the SCAQMD to be cumulatively considerable. This is the reason project-specific and cumulative significance thresholds are the same. Conversely, projects that do not exceed the project-specific thresholds are generally not considered to be cumulatively significant.” The SCAQMD has recognized that there is typically insufficient information to quantitatively evaluate the cumulative contributions of multiple projects because each project applicant has no control over nearby projects. Nevertheless, the potential cumulative impacts from the Project and other projects are discussed below. A cumulative project list was developed for this analysis and is shown in Table 3-11 of the Air Quality study. The Project area is designated as an extreme non‐attainment area for ozone, and a nonattainment area for PM10, PM2.5, and lead. Related projects could contribute to an existing or projected air quality exceedance because the Basin is currently nonattainment for ozone, PM10, and PM2.5. With regard to determining the significance of the contribution from the Project, the SCAQMD recommends that any given project’s potential contribution to cumulative impacts should be assessed using the same significance criteria as for project-specific impacts. Therefore, this analysis assumes that individual projects that do not generate operational or construction emissions that exceed the SCAQMD’s recommended daily thresholds for project-specific impacts would also not cause a commutatively considerable increase in emissions for those pollutants for which the Basin is in nonattainment, and, therefore, would not be considered to have a significant, adverse air quality impact. As previously noted, the Project would not result in any construction-source or operational-source emissions exceedances. Therefore the Project would result in a less than significant impact on a project-specific and cumulative basis. |  |  |  |  |  |  |  |  |  |  |
| d) Expose sensitive receptors to substantial pollutant concentrations? |  |  |  |  |  |  |  |  |  |  |
| The nearest sensitive receptors include March Middle School located immediately to the south and Rainbow Elementary School located 700 feet further to the south on Indian Street and existing single-family homes to the north, west and south. Construction-Source Emissions LST Analysis Table 3-7 identifies the localized impacts at the nearest receptor location in the vicinity of the Project. As shown below, emissions during construction activity would not exceed the SCAQMD’s localized significance thresholds for any criteria pollutant and a less than significant impact would occur. Localized Significance – Long-Tem Operational Activity The proposed project involves the construction and operation of 221 single family residential dwelling units. According to SCAQMD LST methodology, LSTs would apply to the operational phase of a proposed project, if the project includes stationary sources, or attracts mobile sources that may spend long periods queuing and idling at the site (e.g., transfer facilities and warehouse buildings). The proposed project does not include such uses, and thus, due to the lack of significant stationary source emissions, no long-term localized significance threshold analysis is needed. CO “Hot Spot” Analysis The proposed Project considered herein would not produce the volume of traffic required to generate a CO “hot spot” either in the context of the 2003 Los Angeles hot spot study, or based on representative BAAQMD CO threshold considerations, as shown on Table 3-10. Therefore, CO “hot spots” are not an environmental impact of concern for the proposed Project. Localized air quality impacts related to mobile-source emissions would therefore be less than significant. Based upon the conclusions of the project Air Quality study, the project will not expose sensitive receptors to substantial pollutant concentrations. |  |  |  |  |  |  |  |  |  |  |
| e) Create objectionable odors affecting a substantial number of people? |  |  |  |  |  |  |  |  |  |  |
| The Project does not contain land uses typically associated with emitting objectionable odors. Potential odor sources associated with the proposed Project may result from construction equipment exhaust and the application of asphalt and architectural coatings during construction activities and the temporary storage of typical solid waste (refuse) associated with the proposed Project’s (long-term operational) uses. Standard construction requirements would minimize odor impacts from construction. The construction odor emissions would be temporary, short term, and intermittent in nature and would cease upon completion of the respective phase of construction and is thus considered less than significant. It is expected that Project-generated refuse would be stored in covered containers and removed at regular intervals in compliance with the City’s solid waste regulations. The proposed Project would also be required to comply with SCAQMD Rule 402 to prevent occurrences of public nuisances. Therefore, odors associated with the proposed Project construction and operations would be less than significant and no mitigation is required. |  |  |  |  |  |  |  |  |  |  |
| IV. BIOLOGICAL RESOURCES. Would the project: |  |  |  |  |  |  |  |  |  |  |
| a) Have a substantial adverse effect, either directly or through habitat modifications, on any species identified as a candidate, sensitive, or special status species in local or regional plans, policies, or regulations, or by the California Department of Fish and Game or U. S. Fish and Wildlife Service? |  |  |  |  |  |  |  |  |  |  |
| b) Have a substantially adverse effect on any riparian habitat or other sensitive natural community identified in local or regional plans, policies, regulations or by the California Department of Fish and Game or U. S. Wildlife Service? |  |  |  |  |  |  |  |  |  |  |
| (a and b) The project site is bounded on the north, west and south by existing single-family residential development with vacant commercial land to the east which includes a recently approved retail center. Also, the March Middle School and Rainbow Elementary School are located to the south. The site is comprised of level topography and has been disturbed routinely through weed abatement of the site. A Biological Technical report was prepared for the project by Glenn Lukos Associate, Inc. on September 18 2014. This report identifies and evaluates impacts to biological resources associated with the proposed Project in the context of the Western Riverside County Multiple Species Habitat Conservation Plan (MSHCP), the California Environmental Quality Act (CEQA), and State and Federal regulations such as the Endangered Species Act (ESA), Clean Water Act (CWA), and the California Fish and Game Code. |  |  |  |  |  |  |  |  |  |  |
| The Project site is located within the Reche Canyon/Badlands Area Plan of the MSHCP, but is not located within the MSHCP Criteria Area. The Project site is located within the burrowing owl survey area, but is not located within the NEPSSA, CAPSSA, amphibian, or mammal survey areas. Focused burrowing owl surveys were conducted for the Project site; however, no burrowing owls or burrows with owl sign were detected onsite. In compliance with the MSHCP, pre-construction burrowing owl surveys are required prior to site disturbance. The Project site will not impact special-status plants, but will result in the loss of actual or potential habitat for special-status animals, including potential habitat for Stephens’ kangaroo rat (Dipodomys stephensi) [SKR]. Impacts to SKR are covered under the SKR Habitat Conservation Plan (HCP) with payment of the SKR mitigation fee. The loss of potential habitat for other special-status animals would be less than significant due to the low degree of sensitivity of the species, the disturbed nature of the site, and the lack of adjacency to native open space. The Project site does not contain jurisdictional waters, MSHCP riparian/riverine areas, or MSHCP vernal pools. The following discussion provides project-specific mitigation/avoidance measures for actual or potential impacts to special-status resources. Burrowing Owl The Project site contains suitable habitat for burrowing owls; however, burrowing owls were not detected onsite during focused surveys. MSHCP Objective 6 for burrowing owls requires that pre-construction surveys prior to site grading. As such, the following measure is recommended to avoid direct impacts to burrowing owls and to ensure consistency with the MSHCP: BR1. A qualified biologist will conduct a pre-construction presence/absence survey for burrowing owls within 14 days prior to site disturbance. If burrowing owls are detected onsite, the owls will be relocated/excluded from the site outside of the breeding season following accepted protocols, and subject to the approval of the RCA and wildlife agencies. Nesting Birds The Project site contains vegetation with the potential to support nesting birds. As discussed above, the MBTA and California Fish and Game Code prohibit impacts to nesting birds. The following measure is recommended to avoid impacts to nesting birds: BR2. As feasible, vegetation clearing should be conducted outside of the nesting season, which is generally identified as February 1 through September 15. If avoidance of the nesting season is not feasible, then a qualified biologist shall conduct a nesting bird survey within three days prior to any disturbance of the site, including disking, demolition activities, and grading. If active nests are identified, the biologist shall establish suitable buffers around the nests, and the buffer areas shall be avoided until the nests are no longer occupied and the juvenile birds can survive independently from the nests. Therefore, the project as conditioned and subject to the biological resource mitigation measures listed above, will not have a substantial adverse effect, either directly or through habitat modifications, on any species identified as a candidate, sensitive, or special status species in local or regional plans, policies, or regulations, or by the California Department of Fish and Wildlife or U. S. Fish and Wildlife Service. The project will not have a substantially adverse effect on any riparian habitat or other sensitive natural community identified in local or regional plans, policies, regulations or by the California Department of Fish and Wildlife or U. S. Wildlife Service. |  |  |  |  |  |  |  |  |  |  |
| c) Have a substantial adverse effect on federally protected wetlands as defined by Section 404 of the Clean Water Act (including, but not limited to, marsh, vernal pool, coastal, etc.) through direct removal, filling, hydrological interruption, or other means? |  |  |  |  |  |  |  |  |  |  |
| The project site comprised of flat topography. There are no existing trees or vegetation on the project site. The project site is bounded on the north, west and south by existing single-family residential development with vacant commercial land to the east which includes a recently approved retail center. Also to the south are March Middle School and Rainbow Elementary School. Based upon the results of the Biological Technical report prepared for the project, the project site does not contain jurisdictional waters, MSHCP riparian/riverine areas, or MSHCP vernal pools. Therefore, no impacts would occur to federally protected wetlands as defined by Section 404 of the Clean Water Act (including, but not limited to, marsh, vernal pool, coastal, etc.). through direct removal, filling, hydrological interruption, or other means, and no mitigation measures would be required. |  |  |  |  |  |  |  |  |  |  |
| d) Interfere substantially with the movement of any resident or migratory fish or wildlife species or with established native resident migratory wildlife corridors, or impede the use of native wildlife nursery sites? |  |  |  |  |  |  |  |  |  |  |
| The project site comprised of flat topography. There are no existing trees or vegetation on the project site. The project site is bounded on the north, west and south by existing single-family residential development with vacant commercial land to the east which includes a recently approved retail center. Also, the March Middle School and Rainbow Elementary School are located to the south. Based upon the conclusions of the Biological Technical report prepared for this project, there is no evidence of resident or migratory fish or wildlife species was noted on the project site or the adjacent vacant parcel. Therefore, the project will not interfere substantially with the movement of any resident or migratory fish or wildlife species or with established native resident migratory wildlife corridors, or impede the use of native wildlife nursery sites. |  |  |  |  |  |  |  |  |  |  |
| e) Conflict with any local policies or ordinances protecting biological resources, such as a tree preservation policy or ordinance? |  |  |  |  |  |  |  |  |  |  |
| The project site comprised of flat topography. There are no existing trees or vegetation on the project site. The project site is bounded on the north, west and south by existing single-family residential development with vacant commercial land to the east which includes a recently approved retail center. Also, the March Middle School and Rainbow Elementary School are located to the south. There are no existing trees or vegetation on the project site. Therefore related to any local policies or ordinances protecting biological resources, such as a tree preservation policy or ordinance, no impacts would occur and no mitigation measures would be required. |  |  |  |  |  |  |  |  |  |  |
| f) Conflict with the provisions of an adopted Habitat Conservation Plan, Natural Conservation Community Plan, or other approved local, regional, or state habitat conservation plan? |  |  |  |  |  |  |  |  |  |  |
| The project site is not located within one of the Multiple Species Habitat Conservation Plan (MSHCP) criteria areas, which are potential habitat preservation areas. The proposed project will not conflict with the Stephen's Kangaroo Rat Habitat Conservation Plan (SKR HCP) or MSHCP or any other known local, regional or state habitat conservation plans. The project will be conditioned to pay required SKR mitigation fees. Also, the City participates in the MSHCP, a comprehensive habitat conservation-planning program addressing multiple species’ needs, including preservation of habitat and native vegetation in Western Riverside County. This project will also be subject to impact fees to support the implementation of the Multiple Species Habitat Conservation Plan as provided for by City ordinance. |  |  |  |  |  |  |  |  |  |  |
| V. CULTURAL RESOURCES. Would the project: |  |  |  |  |  |  |  |  |  |  |
| a) Cause a substantial adverse change in the significance of a historical resource as defined in Section 15064.5? |  |  |  |  |  |  |  |  |  |  |
| b) Cause a substantial adverse change in the significance of an archaeological resources pursuant to Section 15064.5? |  |  |  |  |  |  |  |  |  |  |
| c) Directly or indirectly destroy a unique paleontological resource or site or unique geologic feature? |  |  |  |  |  |  |  |  |  |  |
| (a, b and c) A Cultural Resource Assessment for the project site was prepared by Helix Environmental Planning April 13, 2016. The cultural resources study included a record search, a Sacred Lands File search, tribal outreach, a review of historic maps and aerial photographs, an intensive survey by a HELIX archaeologist and a Native American monitor, and preparation of a report. The project site is comprised of flat topography with no rock outcroppings or other unique geologic features. Based upon inspections of the project site in November 2015 and again in April 2016 and review of a 1987 citywide survey (Archeological Research Unit, University of California, Riverside), there are no known archaeological resources on the project site. There are no historical structures existing on the project site (General Plan, Figure 5.10-1, Historic Resources Inventory). There are no known historical paleontological or unique geological features on the project site (General Plan, Figures 5.10-2, Prehistoric Sites). Additionally, the City’s Final Program EIR (June 2006), Figure 5.10-3 list the project site as low potential for paleontological sensitive area based on extensive field work (Page 5.10-10). Based on the Cultural Resource Assessment, a record search of the project area and a one-mile radius from the Eastern Information Center (EIC) indicated that eight cultural resources had been recorded within the search radius (see Table 1 of the Cultural Resource Study). One resource (P-33-023936) was mapped within the project property. This resource is a historic period alfalfa farm that encompasses the property adjacent to the project area on the east as well as the southeastern corner of the project area. One feature from this site is located in the project area: the remnants of a grain loading dock from the Barron/Lantz Holdings, tentatively dated for use between 1948 and 1970. This historic feature is not considered a significant resource. The current survey did not identify any cultural resources within the project area other than the previously recorded historic feature, which is not a significant resource. Therefore, no impacts to cultural resources are anticipated. However, the project site is in alluvial soils, where there is a potential for buried cultural resources. Based on this, it is recommended that an archaeological and Native American monitoring program be implemented. The monitoring program would include attendance by the archaeologist and Native American monitor(s) at a preconstruction meeting with the grading contractor and the presence of archaeological and Native American monitors during initial ground-disturbing activities on site. Both archaeological and Native American monitors would have the authority to temporarily halt or redirect grading and other ground-disturbing activity in the event that cultural resources are encountered. The following mitigation measures have been introduced to ensure compliance with City General Plan Policies and the State Public Resources Code: CR-1: Prior to the issuance of a grading permit, the Project Applicant shall provide evidence to the City of Moreno Valley that a professional archaeological monitor has been retained by the Applicant to conduct monitoring of all mass grading and trenching activities and that the monitor has the authority to temporarily halt and redirect earthmoving activities in the event that suspected archaeological resources are unearthed during Project construction. The Project archaeologist, with input from the appropriate Tribe, shall prepare a Cultural Resources Monitoring Plan (CRMP) to document protocols for inadvertent finds, to determine potential protection measures from further damage and destruction for any identified archaeological resource(s)/ tribal cultural resources (TCRs), outline the process for monitoring and for completion of the final Phase IV Monitoring Report. If any archaeological and/or TCRs are identified during monitoring, these will also be documented and addressed per standard archaeological protocols in the Phase IV report, with the exception of human remains which will be addressed per CUL-5. The Project Archaeologist shall attend the pregrading meeting with the City and contractors to explain and coordinate the requirements of the monitoring program. CR-2: At least 30 days prior to the issuance of a grading permit, the Applicant shall contact the appropriate Luiseño tribe to develop a Cultural Resources Treatment Agreement and shall provide evidence to the City of Moreno Valley that the professionally qualified Luiseño Native American monitor(s) has been secured from the interested tribe(s), and that the monitor shall be allowed to monitor all mass grading and trenching activities. The Tribal representative(s) shall attend the pre-grading meeting with the City and contractors to explain and coordinate the requirements of the monitoring program. CR-3: If, during mass grading and trenching activities, the Archaeologist or Tribal representatives suspect that an archaeological resource and/or TCR may have been unearthed, the monitor identifying the potential resources, in consultation with the other monitor as appropriate, shall immediately halt and redirect grading operations in a 100-foot radius around the find to allow identification and evaluation of the suspected resource. The Native American monitor(s) or appropriate representative(s) and the archaeological monitor shall evaluate the suspected resource and make a determination of significance pursuant to California Public Resources Code Section 21083.2. The archaeological monitor and tribal monitor(s) or appropriate representative(s), the Project Applicant, and the City Planning Division shall confer regarding mitigation of the discovered resource(s). All sacred sites, should they be encountered within the Project area, shall be avoided and preserved as the preferred mitigation, if feasible. CR-4: Prior to grading permit issuance, the City shall verify that the following note is included on the Grading Plan: “If any suspected archaeological resources are discovered during ground-disturbing activities and the archaeological monitor or Tribal representatives are not present, the construction supervisor is obligated to halt work in a 100-foot radius around the find and call the project archaeologist and the Tribal representatives to the site to assess the significance of the find." CR-5: If human remains are encountered, California Health and Safety Code Section 7050.5 states that no further disturbance shall occur until the Riverside County Coroner has made the necessary findings as to origin. Further, pursuant to California Public Resources Code Section 5097.98(b), remains shall be left in place and free from disturbance until a final decision as to the treatment and disposition has been made by the Coroner. If the Riverside County Coroner determines the remains to be Native American, the California Native American Heritage Commission must be contacted within 24 hours. The Native American Heritage Commission must then immediately notify the “most likely descendant(s)” of receiving notification of the discovery. The most likely descendant(s) shall then make recommendations within 48 hours, and engage in consultations concerning the treatment of the remains as provided in Public Resources Code §5097.98. |  |  |  |  |  |  |  |  |  |  |
| CR-6: Prior to construction involving excavation four feet or more below existing surface grade, the construction contractor shall provide evidence that a qualified paleontologist has been retained, and that the paleontologist(s) shall be present during all grading and other significant ground-disturbing activities that reach four feet or more below existing surface grade. In the event fossiliferous deposits are encountered, the following measures shall be implemented: - Monitoring shall be conducted by qualified paleontological monitor(s) of excavation in areas identified as likely to contain paleontological resources, including very old alluvial fan deposits. Paleontological monitors shall be equipped to salvage fossils as they are unearthed, to avoid construction delays, and to remove samples of sediments that are likely to contain the remains of small fossil invertebrates and vertebrates. Monitors shall be empowered to temporarily halt or divert equipment to allow removal of abundant or large specimens. Monitoring may be reduced if the potentially fossiliferous units are determined upon exposure and examination by qualified paleontological personnel to have low potential to contain fossil resources. - Paleontological monitoring of any earthmoving will be conducted by a monitor, under direct guidance of a qualified paleontologist. Earthmoving in areas of the parcel where previously undisturbed sediments are buried, but not otherwise disturbed, will not be monitored. - If too few fossil remains are found after 50 percent of the planned-for earthmoving has been completed, monitoring can be reduced or discontinued in those areas at the Project paleontologist’s direction. - Preparation of recovered specimens to a point of identification and permanent preservation, including washing of sediments to recover small invertebrates and vertebrates. - Identification and curation of specimens into a professional, fully accredited museum repository with permanent retrievable storage. The paleontologist must have a written repository agreement in hand prior to the initiation of mitigation activities. - Preparation or a report of findings with and appended itemized inventory of specimens. The report and inventory, when submitted to the city along with confirmation of the curation of recovered of recovered specimens into an established, accredited museum repository, will signify completion of the program to mitigate impacts to paleontological resources. Based on the proceeding information, development of the project will not result in substantial adverse change in the significance of a historical or archaeological resource or result directly or indirectly in the destruction of a unique paleontological resource or site or unique geologic feature. |  |  |  |  |  |  |  |  |  |  |
| d) Disturb any human remains, including those interred outside of dedicated cemeteries? |  |  |  |  |  |  |  |  |  |  |
| No known human remains have been identified at the project site. Compliance with mitigation measure CR-5 as identified in the response to checklist questions a, b, and c for Cultural Resources will also serve to prevent the disturbance of any human remains. |  |  |  |  |  |  |  |  |  |  |
| VI. GEOLOGY AND SOILS. Would the project: |  |  |  |  |  |  |  |  |  |  |
| a) Expose people or structures to potential substantial adverse effects, including the risk of loss, injury or death involving: |  |  |  |  |  |  |  |  |  |  |
| (i) Rupture of a known earthquake fault, as delineated on the most recent Alquist-Priolo Earthquake Fault Zoning Map issued by the State Geologist for the area or based on other substantial evidence of a known fault? Refer to Division of Mines and Geology Special Publication 42. |  |  |  |  |  |  |  |  |  |  |
| According to the City’s General Plan, the project site is not on, or close to, any known earthquake fault. There is no new information that would indicate the existence of a fault or fault tract in proximity of the site. Accordingly, there is no risk of ground rupture due to faulting at the proposed project site. |  |  |  |  |  |  |  |  |  |  |
| (ii) Strong seismic ground shaking? |  |  |  |  |  |  |  |  |  |  |
| According to the City’s General Plan, the project site is not on, or close to, any known earthquake fault. The nearest fault is the San Jacinto fault system, which is located about 8 miles to the northeast. The San Andreas fault system is more than 25 miles from the site. The active Sierra Madre and San Gabriel fault zones lie roughly 35 and 40 miles respectively to the northwest of the site. The active Elsinore and Newport-Inglewood fault zones lie approximately 20 and 45 miles, respectively, to the southwest of the site. This faulting is not considered a significant constraint to development on the site with the use of current building codes. Ground-shaking intensity could be moderately-high during a 100-year interval earthquake. Foundation designs will be reviewed to ensure incorporation of appropriate engineering recommendations to mitigate any such seismicity. There is no new information that would indicate the existence of a fault on the site. |  |  |  |  |  |  |  |  |  |  |
| (iii) Seismic-related ground failure, including liquefaction? |  |  |  |  |  |  |  |  |  |  |
| According to the City’s General Plan, the project site is not on, or close to, any known earthquake fault. However, ground-shaking intensity could be moderately-high during a 100-year interval earthquake. Based on available resources and the City’s General Plan, the potential for seismic related failure or liquefaction on the site is minimal based on the water table and soil conditions at the site. |  |  |  |  |  |  |  |  |  |  |
| (iv) Landslides? |  |  |  |  |  |  |  |  |  |  |
| The project site is not near or adjacent to mountainside areas. Due to a lack of slopes within or nearby the project site seismically induced landslides are not anticipated to pose a danger to the project site. Development of the project will not result in impacts from landslides and no mitigation measures would be required. |  |  |  |  |  |  |  |  |  |  |
| (b) Result in substantial soil erosion or the loss of topsoil? |  |  |  |  |  |  |  |  |  |  |
| The development of the site will likely result in the reduction of erosion with the placement of buildings and landscaping on the site. During construction, there is the potential for less than significant impacts for short-term soil erosion from minimal excavation and grading. This will be addressed as part of standard construction, such as watering to reduce dust and sandbagging, if required, during raining periods. |  |  |  |  |  |  |  |  |  |  |
| (c) Be located on a geologic unit or soil that is unstable, or that would become unstable as a result of the project, and potentially result in on- or off-site landslide, lateral spreading, subsidence, liquefaction or collapse? |  |  |  |  |  |  |  |  |  |  |
| According to the City’s environmental information, the geologic unit or soil is not known to be unstable (Western Riverside Area Soil Survey – University of California Agricultural Experiment Station, 1971). As designed and conditioned, the potential for the impacts resulting from a landslide, lateral spreading, subsidence, liquefaction or collapse is less than significant. |  |  |  |  |  |  |  |  |  |  |
| (d) Be located on expansive soil, as defined in Table 18-1-B of the Uniform Building Code (1994), creating substantial risks to life or property? |  |  |  |  |  |  |  |  |  |  |
| According to the City’s environmental information and the results of a Geotechnical Report prepared by Leighton and Associates, Inc. on June 9, 2004 and an Update prepared on August 29, 2016, project soils evaluated in a near surface sample have a very low expansion potential. The potential for the project to create substantial risks to life or property is less than significant. |  |  |  |  |  |  |  |  |  |  |
| (e) Have soils incapable of adequately supporting the use of septic tanks or alternative waste water disposal systems where sewers are not available for the disposal of waste water? |  |  |  |  |  |  |  |  |  |  |
| The proposed apartment project will operate on a sewer system that will be reviewed, approved and installed according to Eastern Municipal Water District requirements. The proposed project will not be introducing septic tanks or alternative water disposal systems. |  |  |  |  |  |  |  |  |  |  |
| VII. GREENHOUSE GAS EMISSIONS. Would this project? |  |  |  |  |  |  |  |  |  |  |
| a) Generate greenhouse gas emissions, either directly or indirectly, that may have a significant impact on the environment? |  |  |  |  |  |  |  |  |  |  |
| Global climate change is caused by greenhouse gas (GHG) emissions throughout the world. Mitigating global climate change will require worldwide solutions. Greenhouse gases are gases emitted from the earth’s surface that absorb infrared radiation in the atmosphere. Increases in these gases lead to more absorption of radiation and warm the lower atmosphere, and therefore increase evaporation rates and temperatures on the Earth’s surface. The City of Moreno Valley adopted a Climate Action Strategy on October 9, 2012. In 2012, the City of Moreno Valley completed a Greenhouse Gas Analysis (City’s GHG Analysis) that addresses statewide legislation for sustainability through the preparation of GHG inventories and strategies to reduce emissions consistent with AB 32, which established a statewide target to reduce GHG emission to 1990 levels by 2020. The greenhouse gas analysis provides a policy framework for reducing emissions within the City. Following the state’s adopted GHG reduction target, Moreno Valley set a goal to reduce emissions back to 1990 levels by the year 2020. This target was calculated as a 15 percent decrease from 2007 levels. Projects that demonstrate compliance with the reduction target described in the City’s GHG Analysis are considered consistent with the AB 32 reduction target. As provided for in the CEQA Guidelines (Section 15064.4), it is necessary for the lead agency to make a good-faith effort in considering GHG emissions on a project specific basis. A Greenhouse Gas Analysis (GHG) was prepared for the project by Urban Crossroads on November 3, 2016, to analyze potential construction resource and operational resource impacts. |  |  |  |  |  |  |  |  |  |  |
| The City of Moreno Valley has not adopted its own numeric threshold of significance for determining impacts with respect to GHG emissions. The SCAQMD has convened aWorking Group. Based on the last Working Group meeting (Meeting No. 15) held in September 2010, SCAQMD is proposing to adopt a tiered approach for evaluating GHG emissions for development projects where SCAQMD is not the lead agency. SCAQMD had proposed a Project level efficiency significance threshold, in which a 2020 statewide population and employment for land use sectors was divided by 2020 statewide SP, amounting to a 4.8 MTCO2e per service population threshold (1). The City will utilize the Project level efficiency significance threshold approach recommended in the SCAQMD’s Interim Thresholds document for commercial, residential, and mixed use projects. Thus, and based on guidance from the SCAQMD, if a residential project would emit GHGs less than 4.8 MTCO2e per service population, the project is not considered a substantial GHG emitter and the GHG impact is less than significant. On the other hand, if a residential project would emit GHGs in excess of 4.8 MTCO2e per service population, then the project could be considered a substantial GHG emitter, requiring additional analysis and potential mitigation. As identified in Table 4-1 of the Greenhouse Gas Analysis, the proposed project would result in approximately 4.62 MTCO2e per service population and would not exceed the threshold of 4.8 MTCO2e per service population. Therefore, project-related emissions would not have a significant direct or indirect impact on GHG and climate change. |  |  |  |  |  |  |  |  |  |  |
| b) Conflict with an applicable plan, policy or regulation adopted for the purpose of reducing the emissions of greenhouse gases? |  |  |  |  |  |  |  |  |  |  |
| AB 32 is the primary plan, policy or regulation adopted in the State of California to reduce GHG emissions. The City’s GHG Analysis in 2012 addressed statewide legislation for sustainability through the preparation of GHG inventories and strategies to reduce emissions consistent with AB 32, which established a statewide target to reduce GHG emission to 1990 levels by 2020. Preparing a greenhouse gas analysis supports AB 32 at the local level, and the greenhouse gas analysis provides a policy framework for how the City of Moreno Valley can do its part to reduce emissions. The City of Moreno Valley adopted a Climate Action Strategy on October 9, 2012. In 2012, the City of Moreno Valley completed a Greenhouse Gas Analysis (City’s GHG Analysis) that addresses statewide legislation for sustainability through the preparation of GHG inventories and strategies to reduce emissions consistent with AB 32, which established a statewide target to reduce GHG emission to 1990 levels by 2020. Preparing a greenhouse gas analysis supports AB 32 at the local level. The greenhouse gas analysis provides a policy framework for how the City of Moreno Valley can do its part to reduce emissions. Following the state’s adopted GHG reduction target, Moreno Valley set a goal to reduce emissions back to 1990 levels by the year 2020. This target was calculated as a 15 percent decrease from 2007 levels. Projects that demonstrate compliance with the reduction target described in the City’s GHG Analysis are considered consistent with the AB 32 reduction target. The Project is consistent with and supports the City of Moreno Valley Energy Efficiency and Climate Action Strategy (CAS), which is the applicable plan, policy or regulation adopted for the purpose of reducing greenhouse gases. Project consistency with the CAS is detailed in Section 2.10 of the Greenhouse Gas Analysis. Therefore, since the proposed Project meets and exceeds the City’s GHG Analysis reduction target and complies with applicable measures that reduce GHG emissions, the proposed Project will not conflict with any applicable plan, policy or regulation adopted for the purpose of reducing the emissions of greenhouse gases, and impacts in this regard are considered less than significant. |  |  |  |  |  |  |  |  |  |  |
| VIII. HAZARDS AND HAZARDOUS MATERIALS. Would the project? |  |  |  |  |  |  |  |  |  |  |
| a) Create a significant hazard to the public or the environment through the routine transport, use or disposal of hazardous materials? |  |  |  |  |  |  |  |  |  |  |
| The proposed project will not involve the routine transport, use or disposal of hazardous materials. Since the project will not involve the routine transport, use or disposal of hazardous materials, there will be no potential for a significant hazard to the public or the environment. |  |  |  |  |  |  |  |  |  |  |
| b) Create a significant hazard to the public or the environment through reasonably foreseeable upset and accident conditions involving the release of hazardous materials into the environment? |  |  |  |  |  |  |  |  |  |  |
| The proposed project will not involve the routine transport, use or disposal of hazardous materials. The proposed project will not create a significant hazard to the public or the environment through the routine transport, or use or disposal of hazardous materials. Since the project will not involve the routine transport, use or disposal of hazardous materials, there will be no potential for a significant hazard to the public or the environment. |  |  |  |  |  |  |  |  |  |  |
| c) Emit hazardous emissions or handle hazardous or acutely hazardous materials, substances, or waste within one-quarter mile of an existing or proposed school? |  |  |  |  |  |  |  |  |  |  |
| March Middle School is located immediately to the south of the project site and Rainbow Elementary School is located approximately 700 feet further to the south. The project as designed and conditioned will not emit hazardous emissions or handle hazardous materials. |  |  |  |  |  |  |  |  |  |  |
| d) Be located on a site which is included on a list of hazardous materials sites compiled pursuant to Government Code Section 65962.5 and, as a result would it create a significant hazard to the public or the environment? |  |  |  |  |  |  |  |  |  |  |
| The site was checked against the list of hazardous material sites pursuant to Government Code Section 65962.5. The project is not located on a list of hazardous materials sites compiled pursuant to Government Code Section 65962.5. |  |  |  |  |  |  |  |  |  |  |
| e) For a project located within an airport land use plan or, where such a plan has not been adopted, within two miles of a public airport or public use airport, would the project result in a safety hazard for people residing or working in the project area? |  |  |  |  |  |  |  |  |  |  |
| The nearest airport is the March Air Reserve Base located approximately three-quarters of a mile to the west. The distance to the runway is approximately one mile. The project site is located within Compatibility Zone E of the March Air Reserve Base/Inland Port Airport Land Use Compatibility Plan where residential density is not restricted. This project was reviewed by the Riverside County Airport Land Use Commission and in a letter dated May 10, 2016 it was determined to be consistent with the 2014 March Air Reserve Base/Inland Port Airport Land Use Compatibility Plan subject to certain requirements which have been incorporated into the project conditions of approval. The project, as conditioned, will not result in a safety hazard for future residents. |  |  |  |  |  |  |  |  |  |  |
| f) For a project within the vicinity of a private airstrip, would the project result in a safety hazard for people residing or working in the project area? |  |  |  |  |  |  |  |  |  |  |
| There are no private airstrips within the City of Moreno Valley. The project is not within proximity of a private airstrip. Therefore, the project would not result in a safety hazard pertaining to proximity of a private airstrip. |  |  |  |  |  |  |  |  |  |  |
| g) Impair implementation of, or physically interfere with an adopted emergency response plan or emergency evacuation plan? |  |  |  |  |  |  |  |  |  |  |
| The proposed project would not have any direct effect on an adopted emergency response plan, or emergency evacuation plan. The City's emergency plans are also consistent with the General Plan. The proposed project has been designed and conditioned to provide required circulation and required fire access to allow for ingress of emergency vehicles and egress of passenger vehicles. Therefore, the proposed project would not be in conflict in any way with the emergency response or emergency evacuation plans. |  |  |  |  |  |  |  |  |  |  |
| h) Expose people or structures to a significant risk of loss, injury or death involving wildland fires, including where wildlands are adjacent to urbanized areas or where residences are intermixed with wildlands? |  |  |  |  |  |  |  |  |  |  |
| The proposed project site is not adjacent to wildlands and is not located within the Very High Fire Hazard Severity Zone. As designed and conditioned, the project would not expose people or structures to a significant risk of loss, injury or death involving wildland fires. In addition, the project is not located within a designated wildland area. |  |  |  |  |  |  |  |  |  |  |
| IX. HYDROLOGY AND WATER QUALITY. Would the project: |  |  |  |  |  |  |  |  |  |  |
| a) Violate any water quality standards or waste discharge requirements? |  |  |  |  |  |  |  |  |  |  |
| Pursuant to the requirements of the Santa Ana Regional Water Quality Control Board, a project specific Water Quality Management Plan (WQMP) is required of certain projects involving discretionary approval. This project requires a WQMP to address pollutants of concern. Site Design and Source Control best management practices (BMP) are conditioned to be used throughout the project. The project has proposed the use of bioretention facilities and Low Impact Development (LID) BMP’s. Treatment BMPs will be selected and implemented which are medium to highly effective in treating pollutants of concern. Final design and sizing details of all BMPs must be provided in the first submittal of the F-WQMP. The project has been conditioned to provide documentation that runoff will be treated in conformance with the “Riverside County Water Quality Management Plan for Urban Runoff” dated October 22, 2012 and approved by the Santa Ana Regional Water Quality Control Board (Guidance Document). Additionally, grading activities would temporarily expose soils to wind and water erosion that would contribute to downstream sedimentation. The proposed project would comply with all permits and development guidelines associated with urban water runoff and discharge set forth by the City of Moreno Valley and the Regional Water Quality Control Board. With the approval of the storm drainage facilities by the City Engineer and Riverside County Flood Control District (RCFCD), as well as complying with all applicable storm water discharge permits, impacts would be less than significant. |  |  |  |  |  |  |  |  |  |  |
| b) Substantially deplete groundwater supplies or interfere substantially with groundwater recharge such that there would be a net deficit in aquifer volume or a lowering of the local groundwater table level (e.g., the production rate of pre-existing nearby wells would drop to a level which would not support existing land uses or planned uses for which permits have been granted)? |  |  |  |  |  |  |  |  |  |  |
| The Eastern Municipal Water District (EMWD) would provide the proposed project with potable water as opposed to utilizing individual water wells. Potable water is adequate to serve the proposed project. Although the project would cover a majority of the site with impervious surfaces, the landscaped areas would still provide a means for groundwater recharge. Impacts would be less than significant. |  |  |  |  |  |  |  |  |  |  |
| c) Substantially alter the existing drainage pattern of the site or area, including through the alteration of the course of a stream or river, in a manner which would result in substantial erosion or siltation on- or off-site? |  |  |  |  |  |  |  |  |  |  |
| There is no streambed or river on the project site, so the project will not cause a change in the existing on-site drainage pattern that would result in substantial erosion or siltation on- or off-site. During construction of the project, there is the potential for some sediments to be discharged within the storm water system. Erosion control plans are required for projects prior to issuance of grading permits for preventing substantial erosion. The project as designed and conditioned will not change the existing drainage pattern that would result in substantial erosion or siltation on- or off-site. Impacts would be less than significant. |  |  |  |  |  |  |  |  |  |  |
| d) Substantially alter the existing drainage pattern of the site or area, including through the alteration of the course of a stream or river, or substantially increase the rate or surface runoff in a manner which would result in flooding on- or off site? |  |  |  |  |  |  |  |  |  |  |
| There is no streambed or river on the project site. The on-site project storm drain infrastructure proposes to tie into existing storm drain infrastructure in Perris Boulevard. The project will be responsible for completing both and off-site storm drain infrastructure. The project as designed and conditioned will not cause a change in the existing drainage pattern that would result in substantial erosion or siltation on- or off-site. Therefore, project implementation would not result in modifications that could ultimately result in substantial erosion or siltation on- or off-site. Impacts would be less than significant. |  |  |  |  |  |  |  |  |  |  |
| e) Create or contribute runoff which would exceed the capacity of existing or planned stormwater drainage systems or provide substantial additional sources of polluted runoff? |  |  |  |  |  |  |  |  |  |  |
| The proposed project is consistent with the City’s General Plan. All storm drainage improvements would be developed to the standards of the City Engineer and the Riverside County Flood Control and Water Conservation District (RCFCD). Additionally, the project has been designed in accordance with the City’s standard conditions of approval, which includes measures pertaining to storm drainage facilities and runoff. RCFCD provided a letter dated November 2, 2015, indicating that the project site is within the limits of the Sunnymead Master Drainage Plan. The project proposes to construct on-site storm drain infrastructure and detention/water quality basins. Post-construction, the project will not discharge storm water that exceeds historic capacities and will not exceed the capacity of existing or planned stormwater drainage systems. The project will also construct Master Plan Storm Drain system Line M2 and the related utility relocation by constructing approximately 3,000 of off-site storm drain in Santiago Drive, Perris Boulevard, and Iris Avenue. The project will also construct Master Plan Storm Drain system Line D1 by constructing approximately 300 of off-site storm drain in Indian Avenue. As with any urban project, runoff entering the storm drainage system would contain minor amounts of pollutants (including pesticides, fertilizers and motor oil). This would incrementally contribute to the degradation of surface and sub-surface water quality. Additionally, grading activities would temporarily expose soils to water erosion that would contribute to downstream sedimentation. However, the project is subject to the permit requirements of the Santa Ana Regional Water Quality Control Board. As the site is currently unpaved and exposed, development of the proposed project would lessen the existing site contribution to sediment runoff at project completion. Additionally, the approved Preliminary WQMP proposes Best Management Practices for water quality treatment at both the project construction and operational stages. With the approval of the storm drainage facilities by the City Engineer and RCFCD, incorporation of conditions of approval into the project’s design, as well as compliance with all applicable storm water discharge permits, impacts would be less than significant. |  |  |  |  |  |  |  |  |  |  |
| f) Otherwise substantially degrade water quality? |  |  |  |  |  |  |  |  |  |  |
| The proposed project is consistent with the City’s General Plan. All storm drainage improvements would be developed to the standards of the City Engineer and the RCFCD. Additionally, the project has been designed in accordance with the City’s standard conditions of approval, which includes measures pertaining to storm drainage facilities and runoff. As with any urban project, runoff entering the storm drainage system would contain minor amounts of pollutants (including pesticides, fertilizers and motor oil). This would incrementally contribute to the degradation of surface and sub-surface water quality. Additionally, grading activities would temporarily expose soils to water erosion that would contribute to downstream sedimentation. However, the tract is subject to the permit requirements of the Santa Ana Regional Water Quality Control Board. As the site is currently unpaved and exposed, development of the proposed project would lessen the existing site contribution to sediment runoff at project completion. With the approval the storm drainage facilities by the City Engineer and Riverside County Flood Control District, incorporation of conditions of approval into the project’s design, as well as compliance with all applicable storm water discharge permits, impacts would be less than significant. |  |  |  |  |  |  |  |  |  |  |
| g) Place housing within a 100-year floodplain, as mapped on a federal Flood Hazard Boundary or Flood Insurance Rate Map or other flood hazard delineation map? |  |  |  |  |  |  |  |  |  |  |
| h) Place within a 100-year flood hazard area structures which would impede or redirect flood flows? |  |  |  |  |  |  |  |  |  |  |
| (g and h) The proposed project site is located within Federal Emergency Management Agency Zone “X” area outside of the 100-year flood hazard area. This is an area determined to be outside of the 0.2% annual chance flood plain. The project is outside of the delineated dam inundation area for Perris Dam at Lake Perris Reservoir and will not place housing or structures within a 100-year flood hazard area. There are no mountains or steep slopes in proximity to the project site, therefore, there is no chance of mudflows from local mountains. Therefore, impacts would be less than significant. The project as designed and conditioned will not place structures which would impede or redirect flood flows. |  |  |  |  |  |  |  |  |  |  |
| i) Expose people or structures to a significant risk of loss, injury or death involving flooding, including flooding as a result of the failure of a levee or dam? |  |  |  |  |  |  |  |  |  |  |
| The proposed project site is located within Federal Emergency Management Agency Zone “X” area outside of the 100-year flood hazard area. This is an area determined to be outside of the 0.2% annual chance flood plain. The project site is outside of the delineated dam inundation area for Perris Dam at Lake Perris Reservoir and will not expose people or structures to a significant risk of loss, injury or death involving flooding, including flooding as a result of the failure of a levee or dam. |  |  |  |  |  |  |  |  |  |  |
| j) Inundation by seiche, tsunami, or mudflow? |  |  |  |  |  |  |  |  |  |  |
| The project site is not identified in the General Plan as a location subject to seiche, or mudflow. The project is outside of the delineated dam inundation area for Perris Dam at Lake Perris Reservoir. Additionally, due to the position of the proposed project, mudflows from local mountains would be unlikely due to surrounding development. There would be no impacts resulting from inundation by seiche, tsunami, or mudflow. |  |  |  |  |  |  |  |  |  |  |
| X. LAND USE AND PLANNING. Would the project: |  |  |  |  |  |  |  |  |  |  |
| a) Physically divide an established community? |  |  |  |  |  |  |  |  |  |  |
| The project proposes to develop a planned single-family residential community on approximately 53 acres in the R5 zone. Project applications include: - General Plan Amendment from Residential 30 to Residential 5 for a 15.06 acre portion of the approximately 53 acre site; - Zone Change from R30 to R5 for a 15.06 acre portion of the approximately 53 acre site; - Conditional Use Permit for a Planned Unit Development (PUD) to create minimum lot sizes of 4,000 and 5,000 square and unique development standards; and - Tentative Tract Map 36760 to subdivide the approximately 53 acre site into a total of 221 single family residential lots. The project site is bounded by existing single-family tract homes to west and northwest in the RS-10 zone with minimum lot sizes of 4,500 square feet. The property immediate to the north is zoned R5 and has been subdivided with a recorded map, Tract Map 22180. Further to the north are existing single-family tract homes in the R5 zone. Southwest of the project site are single-family homes in the R5 zone. March Middle School and Rainbow Elementary School are located immediately to the south. The California Aqueduct bounds the property along its eastern property line with vacant Community Commercial zoned property to the east. The site to the east was recently approved for development as a Walmart retail center. Additional commercial existing retail centers are located to the southeast at the intersection of Perris Boulevard and Iris Avenue. March Air Reserve Base is located approximately three-quarters of a mile to the west with the City Corporate Yard located approximately 1,400 feet to the east. Since the residential development proposed at this location is an extension of an established land use pattern and is compatible with adjacent General Plan and Zoning districts and existing land uses, the project will not physically divide an established community and impacts would be less than significant under this category. |  |  |  |  |  |  |  |  |  |  |
| b) Conflict with an applicable land use plan, policy or regulation of an agency with jurisdiction over the project (including, but not limited to the general plan, specific plan, local coastal program, or zoning ordinance) adopted for the purpose of avoiding or mitigating an environmental effect? |  |  |  |  |  |  |  |  |  |  |
| This single-family residential project, proposes development that is a permitted use in the R5 zone and are consistent with the goals, objectives and policies of the Residential 5 General Plan designation for the project site. Based upon the conclusions of a Traffic Impact Analysis Report prepared by Urban Crossroads on September 16, 2016, analysis of existing traffic patterns plus project traffic conditions does not result in direct impacts to studied roadway segments or intersections. Analysis of the year 2021 cumulative traffic plus project traffic conditions demonstrated cumulative impacts to three intersections (Indian Street at Cactus Avenue, Indian Street at Gentian Avenue, and Perris Boulevard at Santiago Drive). Mitigation measure requiring the payment of development impact fees and a fair share contribution have been placed on this project to reduce cumulative impacts to less than significant at these three intersections. As designed and conditioned, and subject to implementation of mitigation measures, the project will not conflict with an applicable land use plan, policy or regulation of an agency with jurisdiction over the project including the City’s General Plan. |  |  |  |  |  |  |  |  |  |  |
| c) Conflict with any applicable habitat conservation plan or natural community conservation plan? |  |  |  |  |  |  |  |  |  |  |
| The project is not within one of the Multiple Species Habitat Conservation Plan (MSHCP) criteria areas, which are potential habitat preservation areas. The proposed project will not conflict with the Stephen's Kangaroo Rat Habitat Conservation Plan (SKR HCP) or MSHCP or any other known local, regional or state habitat conservation plans. The project will be conditioned to pay the required SKR mitigation fees. Also, the City participates in the MSHCP, a comprehensive habitat conservation-planning program addressing multiple species’ needs, including preservation of habitat and native vegetation in Western Riverside County. This project will also be subject to fees per City ordinance to support the implementation of the Multiple Species Habitat Conservation Plan. |  |  |  |  |  |  |  |  |  |  |
| XI. MINERAL RESOURCES. Would the project: |  |  |  |  |  |  |  |  |  |  |
| a) Result in the loss of availability of a known mineral resource that would be of value to the region and the residents of the state? |  |  |  |  |  |  |  |  |  |  |
| b) Result in the loss of availability of a locally-important mineral resource recovery site delineated on a local general plan, specific plan or other land use plan? |  |  |  |  |  |  |  |  |  |  |
| (a and b) The project site is located in an urbanized area with additional development occurring in the vicinity. No active mines or mineral recovery programs are currently active within the project site or the surrounding area. Consequently, the development of the project site would not conflict with a mineral recovery plan as adopted by the General Plan. No significant impacts would occur. |  |  |  |  |  |  |  |  |  |  |
| XII. NOISE. Would the project result in: |  |  |  |  |  |  |  |  |  |  |
| a) Exposure of persons to or generation of noise levels in excess of standards established in the local general plan or noise ordinance, or applicable standards of other agencies? |  |  |  |  |  |  |  |  |  |  |
| b) Exposure of persons to or generation of excessive groundborne vibration or groundborne noise levels? |  |  |  |  |  |  |  |  |  |  |
| (a and b) The General Plan Environmental Impact Report (EIR) Noise Section for the City of Moreno Valley states that “The noise generated by construction is addressed by existing city regulations. It is unlawful to create noise that annoys reasonable people of normal sensitivity. The Public Works Department has a standard condition of approval regarding the public nuisance aspect of the construction activities. The construction operations including building related activities and deliveries shall be restricted to Monday through Friday (except for holidays which occur on weekdays), six a.m. to eight p.m.; weekends and holidays (as observed by the city and described in Chapter 2.55 of the Municipal Code), seven a.m. to eight p.m., unless written approval is obtained from the city building official or city engineer. Although construction activities will result in a noise impact, this impact will be short-term and will cease upon completion of construction. The temporary nature of the impact in conjunction with existing city regulations on hours of operation will lessen the potential of a significant impact due to construction noise. However, noise sensitive land use located adjacent to construction sites may be impacted by future construction in the planning area as a result of groundborne noise levels, noise levels that exceed existing standards, and temporary or periodic increases in the ambient noise level. Although not required as mitigation measures to reduce a potentially significant impact to acceptable levels, the following mitigation measures have been introduced to ensure compliance with City General Plan Policies regarding noise: N-1: Construction activities shall be operated in a manner that limits noise impacts on surrounding uses (General Plan Policy 6.5.2). In order to limit noise impacts on surrounding property, the construction contractor will ensure the following: - All construction equipment powered by gasoline or diesel engines will be required to have sound-control devices at least as effective as those originally provided by the manufacturer; no equipment will be permitted to have an unmuffled exhaust. - Mobile noise-generating equipment and machinery will be shut off when not in use; - Construction vehicles assessing the site will be required to use the shortest possible route to and from local freeways, provided the routes do not expose additional receptors to noise. N-2: The staging of construction equipment and the construction trailer shall be placed as far as possible from the existing single-family residences located to the west and south and the schools to the south. The proposed residential development as designed and conditioned is consistent with City Municipal Code development standards and the City’s design guidelines for single-family residential development. It is anticipated that project traffic will operate within acceptable Levels of Service at General Plan build-out, therefore, noise levels will be consistent with General Plan criteria for noise, and noise levels will not exceed the standards set forth in the General Plan. Perceptible groundborne vibrations are typically associated with blasting operations and potentially the use of pile drivers, neither of which will be used during construction of the Proposed Project. As such, no excessive groundborne vibration would be created by the Proposed Project. A less than significant impact would occur. |  |  |  |  |  |  |  |  |  |  |
| c) A substantial permanent increase in ambient noise levels in the project vicinity above levels existing without the project? |  |  |  |  |  |  |  |  |  |  |
| The proposed residential development, as designed and conditioned is consistent with City Municipal Code development standards and Design Guidelines for single-family residential development. Permanent noise associated with the proposed residential development includes, but are not limited to, resident and visitor vehicular traffic, routine landscape and home maintenance, and maintenance of common landscape areas. However, these noise sources would be typical of the adjacent area and therefore, the project would not introduce unique noise sources. Although not required as mitigation measures to reduce a potentially significant impact to acceptable levels, mitigation measures N-1 and N-2 as referenced under Noise checklist questions (a) and (b) have been introduced to ensure compliance with City General Plan Policies related to noise regulation. Therefore, noise levels would be consistent with General Plan criteria for noise, and noise levels will not exceed the standards set forth in the General Plan. Impacts would be less than significant as a result of the proposed project. |  |  |  |  |  |  |  |  |  |  |
| d) A substantially temporary or periodic increase in ambient noise levels in the project vicinity above levels existing without the project? |  |  |  |  |  |  |  |  |  |  |
| During construction, there will be the temporary impact of noise from construction equipment. The nearest sensitive receptors are March Middle School located immediately to the south and Rainbow Elementary School located approximately 700 feet further to the south and existing single-family tract homes to the west and northwest on the west side of Indian Street and to south. The Public Works Department has a standard condition of approval regarding the public nuisance aspect of the construction activities. Any construction within the city shall only be completed between the hours of seven a.m. to seven p.m. Monday through Friday, excluding holidays and from eight a.m. to four p.m. on Saturday, unless written approval is obtained from the city building official or city engineer. According to the Moreno Valley Municipal Code (9.10.030), all temporary construction activities are exempt from the noise standards as long as construction activities are limited to the daytime hours as described above and construction equipment is properly maintained with working mufflers. Although not required as mitigation measures to reduce a potentially significant impact to acceptable levels, mitigation measures N-1 and N-2 as referenced under Noise checklist questions (a) and (b) have been introduced to ensure compliance with City General Plan Policies related to noise regulation. |  |  |  |  |  |  |  |  |  |  |
| e) For a project located within an airport land use plan, or, where such a plan has not been adopted, within two miles of a public airport or public use airport, would the project expose people residing or working in the project area to excessive noise levels? |  |  |  |  |  |  |  |  |  |  |
| The nearest airport is the March Air Reserve Base located approximately three-quarters of a mile to the west. The distance to the runway is approximately one mile. The project site is located within Compatibility Zone E of the March Air Reserve Base/Inland Port Airport Land Use Compatibility Plan. This area is considered a Flight Corridor Buffer zone. Noise impacts are identified as low since the project site is located beyond the 55 CNEL contour. The 55 CNEL contour is below the recommended exterior noise levels caused by aircraft over flight for new residential (65 CNEL) and other development (70 CNEL which requires insulation) as outlined General Plan Policies 6.3.2 and 6.3.3. Noise concerns within Zone E are related to occasional overflights intrusive to some outdoor activities. There are no restrictions on land use or density within this zone. This project was reviewed by the Riverside County Airport Land Use Commission and found to be consistent with the 2014 March Air Reserve Base/Inland Port Airport Land Use Compatibility Plan subject to certain requirements which have been incorporated into the project conditions of approval. |  |  |  |  |  |  |  |  |  |  |
| f) For a project within the vicinity of a private airstrip, would the project expose people residing or working in the project area to excessive noise levels? |  |  |  |  |  |  |  |  |  |  |
| There is no private airstrip within the vicinity of the site, or within the City of Moreno Valley. |  |  |  |  |  |  |  |  |  |  |
| XIII. POPULATION AND HOUSING. Would the project: |  |  |  |  |  |  |  |  |  |  |
| a) Induce substantial population growth in an area, either directly (for example, by proposing new homes and businesses) or indirectly (for example, through extension of roads or other infrastructure)? |  |  |  |  |  |  |  |  |  |  |
| The project site is bounded on the west and south by development with improved street frontage along Indian Street and vacant R5 zone land to the north with a recorded map (Tract Map 22180). Vacant land for commercial (Community Commercial zoning) is located to the east (approved Walmart retail center). The project is surrounded by urban uses. The proposed residential development is consistent with surrounding General Plan land use and Zoning designations. The project will allow for the construction of 221 single-family residential homes. However, the numbers of residential units is less than the total number that would be allowed by the existing General Plan and zoning designations for the site. The project has been conditioned to construct all required on-site and off-site public infrastructure and to participate in the payment of applicable development impact fees. |  |  |  |  |  |  |  |  |  |  |
| b) Displace substantial numbers of existing housing, necessitating the construction of replacement housing elsewhere? |  |  |  |  |  |  |  |  |  |  |
| c) Displace substantial numbers of people, necessitating the construction of replacement housing elsewhere? |  |  |  |  |  |  |  |  |  |  |
| (b and c) This property is currently vacant, and no housing is currently located there. No housing will be displaced by development of this project. The project will not displace any residents. |  |  |  |  |  |  |  |  |  |  |
| XIV. PUBLIC SERVICES. Would the project result in substantial adverse physical impacts associated with the provision of new or physically altered government facilities, need for new or physically altered government facilities, the construction of which could cause significant environmental impacts, in order to maintain acceptable service ratios, response times or other performance objectives for any of the public services: |  |  |  |  |  |  |  |  |  |  |
| a) Fire protection? |  |  |  |  |  |  |  |  |  |  |
| The proposed project has incorporated the City’s standard conditions of approval into its design. These standards specifically address concerns regarding the Fire Prevention Bureau. Standards such as providing approved fire hydrants, fire flow requirements; development impact fee programs and utilizing fire retardant materials have all been incorporated into the project’s design. Insurance Services Office (ISO) ratings are given to firefighting districts in order to rank their operation level. This scale ranges from one (1) the highest possible score, to a ten (10), the worst possible score. The City of Moreno Valley currently has an ISO rating of four (4), which is considered high. With the implementation of the conditions of approval of the project pertaining to Fire Services, impacts would be less than significant. |  |  |  |  |  |  |  |  |  |  |
| b) Police protection? |  |  |  |  |  |  |  |  |  |  |
| The proposed project conforms to the City’s Municipal Code and to the General Plan. Police protection to the project area is provided through the Moreno Valley Police Department. The Police Department was involved in the project review process. Conditions of approval have been included by Police Department to ensure health and safety is protected during construction. Development of the project site would increase the demand for services on the Police Department. The project will pay development impact fees related to Police Facilities. With payment of impact fees, the development of the proposed project would not over-burden their service ability in continuing to provide high quality police service. |  |  |  |  |  |  |  |  |  |  |
| c) Schools? |  |  |  |  |  |  |  |  |  |  |
| d) Parks? |  |  |  |  |  |  |  |  |  |  |
| (c and d) The project would directly increase the use of schools or park facilities through the development of a 221 lot single-family residential planned community. Consistent with City General Plan Policies 4.2.1 and 4.2.14 the City’s Master Plan of Trails and the Master Plan of Parks, this project has been conditioned to construct and then convey to the City a segment of the Juan Bautista De Anza trail within the adjacent California Aqueduct and to construct and convey to the City a public park of approximately 2.0 acres in size with amenities that would include play equipment, a picnic shelter, a gazebo, large group barbeques, concrete picnic tables and benches, concrete waste/recycle containers; drinking fountains, walkway security lighting, decorative concrete walkways, decomposed granite walking path; and tubular steel fencing surrounding the park. The project will pay development impact fees collected and administered by the Moreno Valley Unified School District. |  |  |  |  |  |  |  |  |  |  |
| e) Other public facilities? |  |  |  |  |  |  |  |  |  |  |
| There will be an incremental increase in the demand for new or altered public services including city hall, and city yard facilities. These facilities would be needed with or without the project. This project will be subject to development impact fees, which shall address the impact of the proposed development. |  |  |  |  |  |  |  |  |  |  |
| XV. RECREATION. |  |  |  |  |  |  |  |  |  |  |
| a) Would the project increase the use of existing neighborhood or regional parks or other recreational facilities such that substantial physical deterioration of the facility would occur or be accelerated? |  |  |  |  |  |  |  |  |  |  |
| b) Does the project include recreational facilities or require the construction or expansion of recreational facilities which might have an adverse physical effect on the environment? |  |  |  |  |  |  |  |  |  |  |
| (a and b) As a residential use, the proposed development has the potential to increase the use of parks or other recreational facilities. The proposed project is designed to include private recreational amenities or facilities. The project has also been conditioned to construct and then convey to the City a segment of the Juan Bautista De Anza trail within the adjacent California Aqueduct and to construct and convey to the City a public park of approximately 2.0 acres. |  |  |  |  |  |  |  |  |  |  |
| XVI. TRANSPORTATION/TRAFFIC. Would the project: |  |  |  |  |  |  |  |  |  |  |
| a) Conflict with an applicable plan, ordinance or policy establishing measures of effectiveness for the performance of the circulation system, taking into account all modes of transportation including mass transit and non-motorized travel and relevant components of the circulation system, including but not limited to intersections, streets, highways and freeways, pedestrian and bicycle paths, and mass transit? |  |  |  |  |  |  |  |  |  |  |
| b) Conflict with an applicable congestion management program, including, but not limited to level of service standards and travel demand measures, or other standards established by the county congestion management agency for designated roads or highways? |  |  |  |  |  |  |  |  |  |  |
| (a and b) A Traffic Impact Analysis Report prepared by Urban Crossroads on September 16, 2016, in order to evaluate the proposed General Plan Amendment and Zone Change and the potential circulation system deficiencies that may result from the development of the proposed Project, and to recommend improvements to achieve acceptable circulation system operational conditions. At the City’s direction, the evaluation of General Plan Buildout (2040) traffic conditions was contemplated for the purposes of this TIA. The development of the proposed Project (R5 land use designation) is anticipated to generate 1,799 fewer trip‐ends per day with 135 fewer AM peak hour trips and 156 fewer PM peak hour trips, as compared to the currently adopted General Plan land uses (R5 and R30 land use designation). As such, evaluation of long‐range traffic conditions was determined to be unnecessary as the proposed General Plan Amendment is anticipated to reduce the trips generated by the site. E+P and Opening Year Cumulative traffic conditions have been evaluated as part of this TIA in an effort to identify the near‐term Project impacts, however, long‐range traffic impacts are anticipated to be consistent with or less than those identified by the City’s currently adopted General Plan. Analysis of the year 2021 cumulative traffic plus project traffic conditions demonstrated cumulative impacts to three intersections (Indian Street at Cactus Avenue, Indian Street at Gentian Avenue, and Perris Boulevard at Santiago Drive). A mitigation measure requiring the payment of development impact fees and a fair share contribution have been placed on this project to reduce cumulative impacts to less than significant at these three intersections. Improvement strategies have been recommended at intersections that have been identified as deficient to reduce each location’s peak hour delay and improve the associated LOS grade to an acceptable LOS (LOS D or better). The effectiveness of the proposed recommended improvements is presented in Table 3‐3 for Existing traffic conditions. Recommended improvements to address deficiencies for Existing traffic conditions are described below and analysis worksheets are provided in Appendix 3.4. of the Traffic Impact Analysis Report. The following mitigation measures apply to the project: TR-1: Prior to the issuance of building permits, the Project applicant shall participate in the City’s DIF and County TUMF fee programs by paying the requisite fees at the time of building permit, and in addition pay the Project’s fair share amount of $43,497 for improvements at the intersections of Indian Street at Cactus Avenue and Indian Street at Gentian Avenue as identified in Table 1‐5 that are consistent with the improvements shown on Table 6‐3, or as otherwise agreed to by the City and Project Applicant. Project fair share payment shall only be collected if the City creates a fee program that includes the improvements the fair share contribution is intended to construct. TR-2: Prior to the final approval of the street improvement plans, traffic signal plans will be required for a new traffic signal located at the intersection of Perris Boulevard and Santiago Drive. Prior to issuance of Certificate of Occupancy, the traffic signal and Perris Boulevard and Santiago Drive shall be completed per the approved plans to the satisfaction of the City Engineer. As designed and conditioned, and subject to implementation of mitigation measures TR-1 and TR-2, the project will not conflict with an applicable plan, ordinance or policy establishing measures of effectiveness for the performance of the circulation system and will not conflict with an applicable congestion management program, including, but not limited to level of service standards and travel demand measures, or other standards established by the county congestion management agency for designated roads or highway. |  |  |  |  |  |  |  |  |  |  |
| c) Result in a change in air traffic patterns, including either an increase in traffic levels or a change in location that results in substantial safety risks? |  |  |  |  |  |  |  |  |  |  |
| The nearest airport is the March Air Reserve Base located approximately three-quarters of a mile to the west. The distance to the runway is approximately one mile. The project site is located within Compatibility Zone E of the March Air Reserve Base/Inland Port Airport Land Use Compatibility Plan. This project was reviewed by the Riverside County Airport Land Use Commission and in a letter dated May 10, 2016 it was determined to be consistent with the 2014 March Air Reserve Base/Inland Port Airport Land Use Compatibility Plan subject to specific conditions related to the operation of the project. These conditions of approval have been made a requirement for this development. Therefore, as designed and conditioned, this project will not result in a change in air traffic patterns, including either an increase in traffic levels or a change in location that results in substantial safety risks. |  |  |  |  |  |  |  |  |  |  |
| d) Substantially increase hazards to a design feature (e.g., sharp curves or dangerous intersections) or incompatible uses (e.g. farm equipment)? |  |  |  |  |  |  |  |  |  |  |
| In addition to the project’s on-site public streets, the project has been conditioned by Public Works to complete street improvements where necessary along the site’s Indian Street, Gentian Avenue and Santiago Drive frontages. The street improvements will include but not be limited to, pavement, curb, gutter, sidewalk, streetlights, storm drain, signing and striping, raised median and dry and wet utilities. As designed, the project will not result in hazards, but will help decrease potential hazards at this location. The project is not adjacent to any potential incompatible uses. |  |  |  |  |  |  |  |  |  |  |
| e) Result in inadequate emergency access? |  |  |  |  |  |  |  |  |  |  |
| As designed and conditioned, all driveways and drive aisles will be built to the specifications of the City Engineer and Traffic Engineer, the Fire Prevention Bureau and the General Plan. This will ensure that no hazardous traffic situations would occur during construction or with completion of the project. The site will be readily accessible for emergency access. |  |  |  |  |  |  |  |  |  |  |
| f) Conflict with adopted policies or programs regarding public transit, bicycle, or pedestrian facilities, or otherwise decrease the performance or safety of such facilities? |  |  |  |  |  |  |  |  |  |  |
| The project as designed and conditioned will not conflict with adopted alternative transportation policies, therefore, no adverse impacts would occur. |  |  |  |  |  |  |  |  |  |  |
| XVII. UTILITIES AND SERVICE SYSTEMS. Would the project: |  |  |  |  |  |  |  |  |  |  |
| a) Exceed wastewater treatment requirements of the applicable Regional Water Quality Control Board? |  |  |  |  |  |  |  |  |  |  |
| b) Require or result in construction of new water or wastewater treatment facilities or expansion of existing facilities, the construction of which could cause significant environmental effects? |  |  |  |  |  |  |  |  |  |  |
| (a and b) A Prelminary Water Quality Management Plan (PWQMP) was prepared by Rick Engineering. The PWQMP identifies treatment Best Management Practices (BMP’s) to address the project’s pollutants of concern. The information presented in the PWQMP has been found by the City to be in general conformance with the document, “Water Quality Management Plan for the Santa Ana Region of Riverside County” dated October 22, 2012 and approved by the Santa Ana Regional Water Quality Control Board (Guidance Document). This project will not exceed the wastewater treatment requirements of the Regional Water Quality Control Board. The Eastern Municipal Water District (EMWD) is the sanitary district provider for the project. The project will not exceed wastewater treatment capacity of the Moreno Water Reclamation Facility. |  |  |  |  |  |  |  |  |  |  |
| c) Require or result in the construction of new storm water drainage facilities or expansion of existing facilities, the construction of which could cause significant environmental effects? |  |  |  |  |  |  |  |  |  |  |
| Riverside County Flood Control District (RCFCD) provided a letter dated November 2, 2015, indicating that the project site is within the limits of the Sunnymead Master Drainage Plan. The project proposes to construct Master Plan Storm Drain system Line M2 and the related utility relocation by constructing approximately 3,000 of off-site storm drain in Santiago Drive, Perris Boulevard, and Iris Avenue. The project will also construct Master Plan Storm Drain system Line D1 by constructing approximately 300 of off-site storm drain in Indian Avenue. Construction of the required on and off-site storm drain infrastructure will not cause significant environmental effects. |  |  |  |  |  |  |  |  |  |  |
| d) Have sufficient water supplies available to serve the project from existing entitlements and resources, or are new or expanded entitlements needed? |  |  |  |  |  |  |  |  |  |  |
| The water purveyor, Eastern Municipal Water District (EMWD), prepared an Urban Water Management Plan in 2010 demonstrating that it has or will have sufficient water supplies available to serve urban development within the City of Moreno Valley. EMWD’s plan was based on the City’s General Plan Land Use Element. The proposed residential development is consistent existing General Plan and Zoning designations. Therefore, sufficient water supplies exist to support the proposed project. |  |  |  |  |  |  |  |  |  |  |
| e) Result in a determination by the wastewater treatment provider which serves or may serve the project determined that it has adequate capacity to serve the project’s projected demand in addition to the provider’s existing commitments? |  |  |  |  |  |  |  |  |  |  |
| The wastewater treatment provider is EMWD. The current wastewater treatment facility has adequate capacity to serve projects within Moreno Valley that are consistent with the General Plan and EMWD has plans for major expansions of the Moreno Water Reclamation Facility to serve future needs. Source: EIR for the 2006 General Plan Update. |  |  |  |  |  |  |  |  |  |  |
| f) ) Be served by a landfill with sufficient permitted capacity to accommodate the project’s solid waste disposal needs? |  |  |  |  |  |  |  |  |  |
| Waste Management provides waste hauling service to the City of Moreno Valley. The project will be served by a landfill in the Badlands with sufficient permitted capacity to accommodate the project’s solid waste disposal needs. Source: EIR for the 2006 General Plan Update. |  |  |  |  |  |  |  |  |  |  |
| g) Comply with federal, state, and local statues and regulations related to solid waste? |  |  |  |  |  |  |  |  |  |
| City policies require compliance with State and Federal regulations regarding solid waste. This project will be required to comply with the current policies regarding solid waste. (General Plan Objective 7.8 and Municipal Code Section 6.02) |  |  |  |  |  |  |  |  |  |  |
| XVIII. MANDATORY FINDINGS OF SIGNIFICANCE. |  |  |  |  |  |  |  |  |  |  |
| a) Does the project have the potential to substantially degrade the quality of the environment, substantially reduce the habitat of a fish or wildlife species, cause a fish or wildlife population to drop below self-sustaining levels, threaten to eliminate a plant or animal community, reduce the number or restrict the range of a rare or endangered plant or animal, or eliminate important examples of the major periods of California history or prehistory? |  |  |  |  |  |  |  |  |  |  |
| There are no streambeds or riparian habitat within the project site. There were no surveyed rare plant or animal species noted on the project site. The project would not significantly degrade the quality of the environment or reduce the habitat of a fish or wildlife species, cause a fish or wildlife population to drop below self-sustaining levels, threaten to eliminate a plant or animal community, reduce the number or restrict the range of a rare or endangered plant or animal. There are no historic structures on the site, and there will be no impact to historic resources. The project will not eliminate important examples of the major periods of California history or prehistory. The analysis in this Initial Study demonstrates that project and cumulative impacts would be less than significant. The project as designed and conditioned would not cause substantial adverse health effects on human beings. |  |  |  |  |  |  |  |  |  |  |
| b) Does the project have impacts that are individually limited, but cumulatively considerable? (“Cumulatively considerable” means that the incremental effects of a project are considerable when viewed in connection with the effects of past projects, the effects of other current projects, and the effects of probable future projects)? |  |  |  |  |  |  |  |  |  |  |
| Analysis of the year 2021 cumulative traffic plus project traffic conditions demonstrated cumulative impacts to three intersections (Indian Street at Cactus Avenue, Indian Street at Gentian Avenue, and Perris Boulevard at Santiago Drive). Mitigation measure requiring the payment of development impact fees and a fair share contribution have been placed on this project to reduce cumulative impacts to less than significant at these three intersections. Therefore, this project as conditioned and with mitigation will not create any impacts, that would be considered cumulatively considerable when viewed in connection with existing land uses, other recently approved projects, and existing land use designations. It is not expected that the proposed project would result in incremental effects. The analysis in this Initial Study demonstrates that with the implementation of mitigation measures for cumulative impacts to traffic infrastructure, the proposed project’s cumulative impacts would be less than significant. |  |  |  |  |  |  |  |  |  |  |
| c) Does the project have environmental effects which will cause substantial adverse effects on human beings, either directly or indirectly? |  |  |  |  |  |  |  |  |  |  |
| The project proposes a General Plan Amendment from Residential 30 to Residential 5 and Zone Change from R30 to R5 for a 15.06 acre portion of a 53 acre site in order to develop a 221 lot planned residential community. The project includes applications for Tentative Tract Map 36760 to subdivide the 53 acre site into a total of 221 single family residential lots and a Conditional Use Permit for a Planned Unit Development (PUD). The PUD application will establish minimum lot sizes of 4,000 and 5,000 square feet and establish unique lot widths and setback standards along with architectural guidelines. Analysis of the year 2021 cumulative traffic plus project traffic conditions demonstrated cumulative impacts to three intersections (Indian Street at Cactus Avenue, Indian Street at Gentian Avenue, and Perris Boulevard at Santiago Drive). Mitigation measure requiring the payment of development impact fees and a fair share contribution have been placed on this project to reduce cumulative impacts to less than significant at these three intersections. The project as designed and conditioned and with mitigation will not cause substantial adverse effects on human beings, either directly or indirectly for the reasons described in this checklist/initial study. |  |  |  |  |  |  |  |  |  |  |

List of Key Documents and Resources:

- City of Moreno Valley General Plan, adopted by City Council on July 11, 2006

- City of Moreno Valley Municipal Code, adopted by City Council in 1997

- Preliminary Water Quality Management Plan prepared by Rick Engineering Company

- Traffic Impact Analysis prepared by Urban Crossroads, dated September 13, 2016

- Air Quality Study prepared by Urban Crossroads, dated November 3, 2016

- Greenhouse Gas Analysis prepared by Urban Crossroads, dated November 3, 2016

- Riverside County Integrated Project Long Report, Riverside County Transportation and Land Management Agency,

April 15, 2016

- Western Riverside Area Soil Survey – University of California Agricultural Experiment Station, 1971

- Urban Water Management Plan, Eastern Municipal Water District, 2010

- State Important Farmland Map, 2015, http://maps.conservation.ca.gov/ciff/ciff.html

- Air Quality Management Plan (AQMP), South Coast Air Quality Management Board, 2012

- Cultural Resources Inventory, Archeological Research Unit, University of California, Riverside), October 1987

- Cultural Resource Study prepared by Helix Environmental Planning, dated April 13, 2016

- Geotechnical Report prepared by Leighton and Associates, Inc., dated June 9, 2004

- Update to Geotechnical Report prepared by Leighton and Associates, Inc., dated August 29, 2016

- March Air Reserve Base /Inland Port Airport Land Use Compatibility Plan, Riverside County Airport Land Use Commission, adopted November 13, 2014

- Hydrology Study, prepared by Rick Engineering Company, September 21, 2016

- Flood Insurance Rate Map, Federal Emergency Management Agency, Map Number 06065C0765G, August 28, 2008

- State Wildland Fires Map

- Riverside County Airport Land Use Commission consistency letter dated May 10, 2016

**The above documents and studies are incorporated by reference and available in the case file for Expanded Initial Study PEN16-0096 and the Community Development Department – Planning Division or Public Works Department – Land Development Division.