Mitigation Monitoring Program

Moreno Valley Logistics Center

State Clearinghouse No. 2015061040

Prepared for:

City of Moreno Valley

Community & Economic Development Department

14177 Frederick Street

Moreno Valley, CA  92552

Prepared by:

T&B Planning, Inc.

17542 East 17th Street, Suite 100

Tustin, CA 92780

714-505-6360

Exhibit B to 2017-16

May 2017

# Introduction

## CEQA Requirements

The California Environmental Quality Act (CEQA) requires that when a public agency completes an environmental document that includes measures to mitigate or avoid significant environmental effects, the public agency must adopt a Mitigation Monitoring Program (MMP) for the changes to the project that it has adopted or made a condition of project approval in order to mitigate or avoid significant environmental impacts.  The appropriate reporting or monitoring plan must be designed to ensure compliance during project implementation (Public Resources Code § 21081.6).

The City of Moreno Valley & Economic Community Development Department, Planning Division, would coordinate the monitoring of the mitigation measures with each applicable City department or division, while various City departments/divisions would be responsible for monitoring and verifying compliance of specific mitigation measures (see the Mitigation Monitoring and Reporting Summary Table beginning on page 6).  The City’s Public Works Department would coordinate monitoring of the implementation of all mitigation measures for the project.  Monitoring will include: 1) verification that each mitigation measure has been implemented; 2) recordation of the actions taken to implement each mitigation measure; and 3) retention of records in the project file.

## Program Objectives

The objectives of the MMP for the proposed Moreno Valley Logistics Center Project (the “Project”) include the following:

To provide assurance and documentation that mitigation measures are implemented as planned;

To collect analytical data to assist City administration in its determination of the effectiveness of the adopted mitigation measures;

To report periodically regarding project compliance with mitigation measures, performance standards and/or other conditions; and

To make available to the public, upon request, the City record of compliance with project mitigation measures.

## Overview of the Project

The approximately 89.4-acre site is located in the southern portion of the City of Moreno Valley.  The City of Moreno Valley is located in the northwestern portion of Riverside County, California, and is north of the City of Perris and southeast of the City of Riverside.  The Project site is located approximately 1.3 miles east of Interstate 215 (I-215), 4.2 miles south of State Route 60 (SR-60), and approximately 2.5 miles northwest of Lake Perris.  At a local scale, the Project site is located south of Krameria Avenue, north of Cardinal Avenue, east of Heacock Street and the March Air Reserve Base, and west of Indian Street.

The proposed Project involves the construction and operation of a logistics center with four (4) buildings and a combined 1,736,180 square feet (s.f.) of total floor space.  No future building occupants are yet identified, but the types of occupants are anticipated to include high cube warehousing in the largest building and uses such as general warehousing, industrial, manufacturing, assembly, e-commerce, and similar use types in the smaller buildings.  Associated improvements to the Project site would include, but not be limited to, surface parking areas, vehicle drive aisles, truck courts, utility infrastructure, landscaping, exterior lighting, signage, and water quality/detention basins.  The Project also would construct frontage improvements to Krameria Avenue, Heacock Avenue, and Indian Street, and construct storm drain outlets to the Perris Valley Storm Drain Channel, a segment of which transects the site.

A list of the primary actions under City jurisdiction, as well as other discretionary and administrative actions that would or could be necessary to implement the Project are listed below.

| Matrix of Project Approvals/Permits |  |
| --- | --- |
| Public Agency | Approvals and Decisions |
| City of Moreno Valley |  |
| Proposed Project – City of Moreno Valley Discretionary Approvals |  |
| City of Moreno Valley Planning Commission | - Provide recommendations to the City of Moreno Valley City Council whether to approve the Specific Plan Amendment P15-036, Tentative Parcel Map No. 36150 (PA15-0018), and Plot Plans PA15-0014, PA15-0014, PA15-0015, and PA15-0016. - Provide recommendations to the City of Moreno Valley City Council regarding certification of this EIR. |
| City of Moreno Valley City Council | - Approve, conditionally approve, or deny Specific Plan Amendment No. P15-036. - Approve, conditionally approve, or deny Tentative Parcel Map No. 36150 (PA15-0018). - Approve, conditionally approve, or deny Plot Plan PA15-0014. - Approve, conditionally approve, or deny Plot Plan PA15-0015. - Approve, conditionally approve, or deny Plot Plan PA15-0016. - Approve, conditionally approve, or deny Plot Plan PA15-0017. - Reject or certify this EIR along with the appropriate CEQA Findings (P15-037) |
| Subsequent City of Moreno Valley Discretionary and Ministerial Approvals |  |
| City of Moreno Valley Implementing Approvals | - Approve Final Maps, parcel mergers, lot line adjustments or parcel consolidations, as may be appropriate. - Approve Conditional or Temporary Use Permits, if required. - Issue Grading Permits. - Issue Building Permits. - Approve Road Improvement Plans. - Issue Encroachment Permits. - Approve Street Vacations. - Accept public-right-of way dedications. - Approvals by Moreno Valley Utility associated with removing, relocating, and installing electrical infrastructure. |
| Other Agencies – Subsequent Approvals and Permits |  |
| Riverside County Water Flood Control and Water Conservation District | - Approvals for on- and off-site drainage infrastructure. - Issuance of a Water Quality Management Permit. |
| Eastern Municipal Water District | - Approvals for the construction of on and off-site water and sewer infrastructure. |
| Santa Ana Regional Water Quality Control Board | - Issuance of a Section 401 Permit. - Issuance of a Construction Activity General Construction Permit. - Issuance of a National Pollutant Discharge Elimination System (NPDES) Permit. |
| California Department of Fish and Wildlife | - Issuance of a Lake and Streambed Alteration agreement. |
| United States Army Corps of Engineers | - Issuance of a Section 404 Permit. |
| Riverside County Airport Land Use Commission | - Determination of consistency with the ALUCP. |
| Federal Emergency Management Agency | - Approval of Conditional Letter of Map Revision (CLOMR) and Letter of Map Revision (LOMR) to revise Flood Insurance Rate Map. |

## Organization of the Mitigation Monitoring Program

The following describes the sections of this MMP:

Introduction - Provides an overview of CEQA’s monitoring and reporting requirements, program objectives, the project for which the program has been prepared, and the manner in which this MMP is organized.

MMP - Describes the City entities responsible for implementation of the mitigation monitoring plan, the plan scope, procedures for monitoring and reporting, public availability of documents, the process for making changes to the program, types of mitigation measures, and the manner in which monitoring will be coordinated to ensure implementation of mitigation measures.

Mitigation Monitoring and Reporting Summary - Outlines the Project’s environmental effects and mitigation measures, responsible entities, and the timing for monitoring and reporting for each mitigation measure included in this MMP.

# Description of Plan

## Mitigation Monitoring Plan

This MMP delegates responsibilities for monitoring the project, and allows responsible City entities flexibility and discretion in determining how best to monitor implementation.  Monitoring procedures will vary according to the type of mitigation measure.  The timing for monitoring and reporting is described in the monitoring and reporting summary table, below.  Adequate monitoring requires demonstration of monitoring procedures and implementation of mitigation measures.

In order to enhance the effectiveness of the monitoring program, the City will utilize existing systems where appropriate.  For instance, with any major construction project, the administration generally has at least one inspector assigned to monitor project construction.  These inspectors are familiar with a broad range of regulatory issues and will provide first line oversight for much of the monitoring program.

Responsibilities of the City include identification of typical mitigation measure-related issues such as noisy equipment, dust, safety problems, etc.  Any problems are generally corrected through directions to the contractors or through other appropriate, established mechanisms.  Internal reporting procedures are already in place to document any problems and to address broader implementation issues.

## Reporting Procedures

The City will be responsible for monitoring and implementing the mitigation measures included in this monitoring plan.  Reporting establishes a record that a mitigation measure is being implemented and generally involves the following steps:

The City distributes reporting forms to the appropriate City Departments (as indicated on the Mitigation Monitoring and Reporting forms) or employs the office’s existing reporting process for verification of compliance.

Responsible entities verify compliance by signing the monitoring and reporting form and/or documenting compliance using their own internal procedures when monitoring is triggered.

Responsible entities provide the City with verification that monitoring has been conducted and ensure, as applicable, that mitigation measures have been implemented.

The reporting forms prepared by the City will document the implementation status of mitigation measures of the Project.  Progress reports describe the monitoring status of all mitigation measures.  The City will keep records of Project reporting forms and periodic status reports.

The City also is responsible for assisting the Project’s contractor(s) with reporting responsibilities to ensure that they understand their charge and complete their reporting procedures accurately and on schedule.

## Public Availability

All monitoring reporting forms, summaries, data sheets, and correction instructions related to this MMP for Moreno Valley Logistics Center will be available for public review upon request at the Community Development Department, Planning Division of the City of Moreno Valley (14177 Frederick St., Moreno Valley, CA 92553) during normal business hours.

## Program Changes

If minor changes are required to this MMP, they will be made in accordance with CEQA and would be permitted after further review by the City.  Such changes could include reassignment of monitoring and reporting responsibilities and/or minor modifications to mitigation measures that achieve the same or better end results.  No change will be permitted unless the Mitigation Monitoring Program continues to satisfy the requirements of Public Resources Code §21081.6.

## Types of Mitigation Measures Being Monitored

The Final Environmental Impact Report for the Moreno Valley Logistics Center Project is a “project specific” and “cumulative” evaluation as defined in the CEQA Guidelines.

The Final Environmental Impact Report recommends 67 project specific and cumulative mitigation measures to reduce impacts related to aesthetics, air quality, biological resources, cultural resources, greenhouse gas emissions, hazards and hazardous materials, hydrology and water quality, land use and planning, noise, and transportation/traffic.  Compliance with these mitigation measures will be accomplished through administrative controls over project planning and implementation.  Monitoring would be accomplished as described previously under “Reporting Procedures” through verification and certification by personnel.

In general, implementation of the MMP will require the following actions:

Appropriate mitigation measures would be included in construction documents.

Departments with reporting responsibilities would review the Final Environmental Impact Report, which provides general background information on the reasons for including specified mitigation measures.

Problems with or exceptions to compliance would be addressed by the City as appropriate.

Periodic meetings may be held during project implementation to report on compliance with mitigation measures.

Mitigation Monitoring and Reporting Summary

| Mitigation Measure (MM) | MM is applicable to which building(s) | Responsible Party | Verification of Compliance | Timing | Start Date | Finish Date | Monitoring |  |
| --- | --- | --- | --- | --- | --- | --- | --- | --- |
|  |  |  |  |  |  |  | Date | Monitor |
| Aesthetics |  |  |  |  |  |  |  |  |
| MM 4.1-1 In the event that solar panels are proposed for installation, then prior to the issuance of building permits the City of Moreno Valley shall review the construction drawings and ensure that: All solar panels shall be installed at a fixed angle (i.e., non-tracking); All solar panels shall contain a non-reflective coating or shall be otherwise designed, engineered, and/or installed to minimize glare; and All solar panels shall be directed toward the sky and not facing adjacent properties. | All | Project Applicant | City of Moreno Valley Planning Division & Building and Safety Division | Prior to building permit issuance |  |  |  |  |
| MM 4.1-2 In the event that solar panels are proposed for installation, then prior to the issuance of building permits the Project Applicant shall provide the City of Moreno Valley with evidence that the proposed solar array(s) would not result in substantial glare effects to operations at the March Air Reserve Base as determined by Sandia National Laboratories’ Solar Glare Hazard Analysis Tool (or equivalent analytical model) and to the satisfaction of the Riverside County Airport Land Use Commission. | All | Project Applicant | City of Moreno Valley Planning Division & Building and Safety Division | Prior to building permit issuance |  |  |  |  |
| Air Quality |  |  |  |  |  |  |  |  |
| MM 4.3-1 Prior to building permit issuance, the City of Moreno Valley shall verify that a note is provided on all building plans specifying that compliance with SCAQMD Rule 1113 is mandatory during the application of architectural coatings. Project contractors shall be required to comply with the note and maintain written records of such compliance that can be inspected by the City of Moreno Valley upon request. This note also shall indicate that only “low-volatile organic compound” paint products (no more than 50 gram/liter of VOC) shall be used. All other architectural coatings shall comply with the VOC limits prescribed by SCAQMD Rule 1113. | All | Project Applicant; Project Construction Contractors | City of Moreno Valley Planning Division & Building and Safety Division | Prior to building permit issuance. |  |  |  |  |
| MM 4.3-2 During construction activities, the construction contractor shall maintain a list of diesel-powered construction equipment used on the site, including type/engine year of equipment, number of equipment, and equipment horsepower. The construction contractor shall also maintain a log of the daily operating hours of each piece of diesel-powered equipment by horsepower hours. The construction contractor shall ensure that the usage of diesel-powered construction equipment does not exceed the horsepower-hours per day specified below. Lower tier types may be substituted for higher tier types. Tier 0 – 3,608 horsepower-hours/day Tier 1 – 7,760 horsepower-hours/day Tier 2 – 1,760 horsepower-hours/day Tier 3 – 11,128 horsepower-hours/day Tier 4 – 37,008 horsepower-hours/day | All | Project Applicant; Project Construction Contractors | City of Moreno Valley Building and Safety Division | During construction activities. |  |  |  |  |
| MM 4.3-3 The Project shall comply with California Code of Regulations Title 13, Division 3, Chapter 1, Article 4.5, Section 2025, “Regulation to Reduce Emissions of Diesel Particulate Matter, Oxides of Nitrogen and Other Criteria Pollutants, from In-Use Heavy-Duty Diesel-Fueled Vehicles” and California Code of Regulations Title 13, Division 3, Chapter 10, Article 1, Section 2485, “Airborne Toxic Control Measure to Limit Diesel-Fueled Commercial Motor Vehicle Idling” by complying with the following requirements. To ensure and enforce compliance with these requirements and thereby limit the release of diesel particulate matter, oxides of nitrogen, and other criteria pollutants into the atmosphere from the burning of fuel, prior to grading permit and building permit issuance, the City of Moreno Valley shall verify that the following notes are included on the grading and building plans. Project construction contractors shall be required to ensure compliance with the notes and permit periodic inspection of the construction site by City of Moreno Valley staff or its designee to confirm compliance. These notes also shall be specified in bid documents issued to prospective construction contractors. - Temporary signs shall be placed on the construction site at all construction vehicle entry points and at all loading, unloading, and equipment staging areas indicating that heavy duty trucks and diesel powered construction equipment are prohibited from idling for more than three (3) minutes. The signs shall be installed before construction activities commence and remain in place during the duration of construction activities at all loading, unloading, and equipment staging areas. Construction vehicles shall use the City’s designated truck route. Construction parking shall be located and configured to minimize traffic interference on public streets. Temporary traffic controls such as a flag person shall be used at Project site construction entrances. A construction management plan shall be designed to minimize the number of large construction equipment operating during any given time period. To the extent feasible, construction truck trips shall be scheduled during non-peak hours to reduce peak hour emissions. CARB certified equipment shall be used for construction activities to the extent feasible. Contractors shall be required to turn off all construction equipment and delivery vehicles when not in use and/or idling in excess of 3 minutes. Construction equipment engine sizes shall be limited to the minimum practical size. Electrical powered equipment shall be utilized in-lieu of gasoline-powered engines where technically feasible. Temporary traffic controls, such as a flag person shall be provided during all phases of construction to maintain smooth traffic flow. Construction tucks shall be routed away from congested streets and sensitive receptor areas. Construction parking areas shall be configured to minimize traffic interference. Construction worker trips shall be reduced by encouraging carpooling and providing on-site food service options for the construction crew. Construction workers shall be encouraged to utilize shuttle service to transit stations/multimodal center. | All | Project Applicant; Project Construction Contractors | City of Moreno Valley Land Development Division. Building and Safety Division, and Planning Division | Prior to the issuance of a grading permits and building permits. |  |  |  |  |
| MM 4.3-4 The Project shall comply with the provisions of South Coast Air Quality Management District Rule 403, “Fugitive Dust.” Rule 403 requires implementation of best available dust control measures during construction activities that generate fugitive dust, such as earth moving, grading, and equipment travel on unpaved roads. Prior to grading permit issuance, the City of Moreno Valley shall verify that the following notes are specified on the grading plan. Project construction contractors shall be required to ensure compliance with the notes and permit periodic inspection of the construction site by City of Moreno Valley staff or its designee to confirm compliance. These notes shall also be specified in bid documents issued to prospective construction contractors. - During grading and ground-disturbing construction activities, the construction contractor shall ensure that all unpaved roads, active soil stockpiles, and areas undergoing active ground disturbance within the Project site are watered at least three (3) times daily during dry weather. Watering, with complete coverage of disturbed areas by water truck, sprinkler system, or other comparable means, shall occur in the mid-morning, afternoon, and after work is done for the day. The contractor or builder shall designate a person or persons to monitor the dust control program and to order increased watering, as necessary, to prevent transport of dust offsite. Temporary signs shall be installed on the construction site along all unpaved roads indicating a maximum speed limit of 15 miles per hour (MPH). The signs shall be installed before construction activities commence and remain in place for the duration of construction activities that include vehicle activities on unpaved roads. Gravel pads must be installed at all access points to prevent tracking of mud onto public roads. Install and maintain trackout control devices in effective condition at all access points where paved and unpaved access or travel routes intersect (eg. Install wheel shakers, wheel washers, and limit site access.) Limit fugitive dust sources to 20 percent opacity. When materials are transported off-site, all material shall be covered or effectively wetted to limit visible dust emissions, and at least six inches of freeboard space from the top of the container shall be maintained. All street frontages adjacent to the construction site shall be swept at least once a day using SCAQMD Rule 1186 certified street sweepers utilizing reclaimed water trucks if visible soil materials are carried to adjacent streets. Post a publicly visible sign with the telephone number and person to contact regarding dust complaints. This person shall respond and initiate corrective action within 24 hours. Any vegetative cover to be utilized onsite shall be planted as soon as possible to reduce the disturbed area subject to wind erosion. Irrigation systems required for these plants shall be installed as soon as possible to maintain good ground cover and to minimize wind erosion of the soil. Any on-site stock piles of debris, dirt, or other dusty material shall be covered or watered as necessary to minimize fugitive dust pursuant to SCAQMD Rule 403. A high wind response plan shall be formulated for enhanced dust control if winds are forecast to exceed 25 mph in any upcoming 24-hour period. | All | Project Applicant; Project Construction Contractors. | City of Moreno Valley Planning Division, Building and Safety Division, and Land Development Division | Prior to the issuance of a grading permit. |  |  |  |  |
| MM 4.3-5 The Project shall comply with the provisions of South Coast Air Quality Management District Rule 1186 “PM10 Emissions from Paved and Unpaved Roads and Livestock Operations” and Rule 1186.1, “Less-Polluting Street Sweepers” by complying with the following requirements. To ensure and enforce compliance with these requirements, prior to grading and building permit issuance, the City of Moreno Valley shall verify that the following notes are included on the grading and building plans. Project construction contractors shall be required to ensure compliance with the notes and permit periodic inspection of the construction site by City of Moreno Valley staff or its designee to confirm compliance. The notes also shall be specified in bid documents issued to prospective construction contractors. - If visible dirt or accumulated dust is carried onto paved roads during construction, the contractor shall remove such dirt and dust at the end of each work day by street cleaning. Street sweepers shall be certified by the South Coast Air Quality Management District as meeting the Rule 1186 sweeper certification procedures and requirements for PM10-efficient sweepers. All street sweepers having a gross vehicle weight of 14,000 pounds or more shall be powered with alternative (non-diesel) fuel or otherwise comply with South Coast Air Quality Management District Rule 1186.1. | All | Project Applicant; Project Construction Contractors | City of Moreno Valley Land Development Division, Building and Safety Division, and Planning Division | Prior to the issuance of grading permits and building permits. |  |  |  |  |
| - MM 4.3-6 The Project shall comply with the provisions of SCAQMD Rule 431.2, “Sulfur Content of Liquid Fuels” by complying with the following requirement. To ensure and enforce compliance with this requirement and thereby limit the release of sulfur dioxide (SOX) into the atmosphere from the burning of fuel, prior to grading and building permit issuance, the City of Moreno Valley shall verify that the following note is included on the grading and building plans. Project contractors shall be required to ensure compliance with this note and permit periodic inspection of the construction site by City of Moreno Valley staff or its designee to confirm compliance. This note also shall be specified in bid documents issued to prospective construction contractors. - a) All liquid fuels shall have a sulfur content of not more than 0.05 percent by weight, except as provided for by South Coast Air Quality Management District Rule 431.2. | All | Project Applicant; Project Construction Contractors | City of Moreno Valley Land Development Division & Building and Safety Division | Prior to issuance of grading permits and building permits. |  |  |  |  |
| MM 4.3-7 All indoor forklifts used in the Project’s buildings shall be electric, natural gas, or propane powered. This requirement shall be noted in the buildings’ sale and lease agreements and also shall be included on all tenant improvement plans submitted to the City of Moreno Valley. | All | Project Applicant | City of Moreno Valley Planning Division | Prior to issuance of occupancy permits. |  |  |  |  |
| MM 4.3-8 All outdoor cargo handling equipment (including yard trucks, hostlers, yard goats, pallet jacks, forklifts, and other on-site equipment) that are powered by diesel fuel shall comply with the CARB/ U.S. EPA Tier IV Engine standards for off-road vehicles or better (defined as less than or equal to 0.015 g/bhp-hr. for PM10). This requirement shall be noted in the buildings’ sale and lease agreements and also shall be noted on all tenant improvement plans. | All | Project Applicant | City of Moreno Valley Planning Division | Prior to issuance of occupancy permits. |  |  |  |  |
| MM 4.3-9 Prior to the issuance of a building permit, documentation shall be provided to the City of Moreno Valley demonstrating that: 1) the building is designed to achieve efficiency equal to or exceeding the 2013 California Title 24 Energy Efficiency Standards and complies with the mandatory reductions in indoor water usage required by the California Building Standards Code, including the use of U.S. EPA Certified WaterSense labeled or equivalent faucets, high-efficiency toilets, and water-conserving shower heads; and 2) the landscaping design uses a plant palette emphasizing drought-tolerant plants and use of water-efficient irrigation techniques. | All | Project Applicant | City of Moreno Valley Planning Division & Building and Safety Division | Prior to the issuance of building permits. |  |  |  |  |
| MM 4.3-10 Prior to building final, documentation shall be provided to the City of Moreno Valley demonstrating the appliances and fixtures installed in restrooms and employee break areas are Energy Star rated and/or are U.S. EPA WaterSense labeled or equivalent. | All | Project Applicant | City of Moreno Valley Planning Division & Building and Safety Division | Prior to building final. |  |  |  |  |
| MM 4.3-11 Legible, durable, weather-proof signs shall be placed at truck access gates, loading docks, and truck parking areas that identify applicable California Air Resources Board (CARB) anti-idling regulations. At a minimum each sign shall include: 1) instructions for truck drivers to shut off engines when not in use; 2) a prohibition on the idling of trucks for more than three (3) minutes; 3) instructions for truck drivers to shut down engines after 300 seconds of continuous idling operation once the vehicle is stopped, the transmission is set to “neutral” or “park” and the parking break is engaged; and 4) telephone numbers of the building facilities manager and the CARB to report violations. Prior to building final, the City of Moreno Valley shall conduct a site inspection to ensure that the signs are in place. | All | Project Applicant | City of Moreno Valley Planning Division & Building and Safety Division | Prior to building final. |  |  |  |  |
| MM 4.3-12 Prior to building final, the City of Moreno Valley shall verify that: 1) the parking lot striping and security gating plan allows for adequate truck stacking at gates to prevent queuing of trucks outside the property; and 2) preferential parking locations are identified on the site for carpool, vanpool, EVs and CNG vehicles; and 3) secure, weather protected bicycle parking is provided for building employees. | All | Project Applicant | City of Moreno Valley Planning Division & Transportation Division | Prior to building final. |  |  |  |  |
| MM 4.3-13 Prior to the issuance of building final, the Project’s property owner shall provide a model lease agreement to the Planning Division verifying that provisions are included in the building’s lease agreement that inform tenants about the availability of the following and their benefits to air quality: 1) alternatively fueled cargo handling equipment; 2) grant programs for diesel fueled vehicle engine retrofit and/or replacement; 3) designated truck parking locations in the City of Moreno Valley; 4) access to alternative fueling stations in the City of Moreno Valley that supply compressed natural gas (closest station is located on Indian Street, south of Nandina Avenue); 5) the United States Environmental Protection Agency’s SmartWay program; and 6) voluntary trip reduction programs, for which all employees shall be eligible to participate. | All | Project’s Property Owner | City of Moreno Valley Planning Division & Building and Safety Division | Prior to the issuance of building final. |  |  |  |  |
| MM 4.3-14 Prior to the issuance of building final, the Project’s property owner shall provide a model lease agreement to the Planning Division verifying that provisions are included in the building’s lease agreement that encourages: 1) all fleet vehicles to conform to 2010 air quality standards or better; users shall maintain compliance through normal course of business; and 2) use of electrical equipment for landscape maintenance to the extent feasible; 3) use of electrical powered equipment in lieu of gasoline-powered engines where technically feasible; and 4) reduced-fee or no-fee parking for EVs and CNG vehicles. | All | Project Property Owner(s) | City of Moreno Valley Planning Division & Building and Safety Division | Prior to the issuance of building final |  |  |  |  |
| MM 4.3-15 Prior to the issuance of building final, the Project’s property owner shall provide a model lease agreement to the Planning Division verifying that provisions will be included in the building’s lease agreement that 1) encourages tenants to display information about alternative transportation options in a common area of the building and 2) informs tenants about locations of the nearest existing and planned Metrolink stations and the benefits of implementing a voluntary carpool or rideshare program for employees. | All | Project Property Owner(s) | City of Moreno Valley Planning Division & Building and Safety Division | Prior to the issuance of building final. |  |  |  |  |
| MM 4.3-16 The building plans shall include conduit and plug-in locations for electric yard tractors, fork lifts, reach stackers, and sweepers. | All | Project Applicant | City of Moreno Valley Planning Division & Building and Safety Division | Prior to the issuance of building permits. |  |  |  |  |
| MM 4.3-17 Prior to building final, the City of Moreno Valley shall verify that a sign has been installed at each exit driveway, providing directional information to the City’s truck route. Text on the sign shall read “To Truck Route” with a directional arrow. | Buildings 1 and 2 | Project Applicant | City of Moreno Valley Planning Division & Building and Safety Division | Prior to building final. |  |  |  |  |
| MM 4.3-18 Prior to the issuance of a building permit for any building that utilizes refrigerated storage, any spaces utilizing refrigerated storage shall provide an electrical hookup for refrigeration units on delivery trucks. As a condition of occupancy permits, trucks incapable of utilizing the electrical hookup for powering refrigeration shall be prohibited from accessing the site. | All | Project Applicant | City of Moreno Valley Planning Division & Building and Safety Division | Prior to the issuance of building permits. |  |  |  |  |
| MM 4.3-19 Prior to the issuance of building permits, to ensure the shading of parking lots to reduce solar gain, the City of Moreno Valley shall review landscaping plans to verify that the plans call for the planting of shade trees so that at least 50% of the automotive parking lots (excluding the truck courts where trees cannot be planted due to interference with truck maneuvering) will be shaded within 15 years after Project construction is complete. | All | Project Applicant | City of Moreno Valley Planning Division & Building and Safety Division | Prior to the issuance of building permits. |  |  |  |  |
| MM 4.3-20 The Project is required to comply with the provisions of SCAQMD Rule 402 “Nuisance.” To ensure and enforce compliance with this requirement, which applies to the release of odorous emissions into the atmosphere, prior to the issuance of grading and building permits, the City of Moreno Valley shall verify that the following note is included on grading and building plans. During Project construction, contractors shall be required to ensure compliance with Rule 402 and permit periodic inspection of the construction site by the City of Moreno Valley staff or its designee to confirm compliance. The note shall be specified in bid documents issued to prospective construction contractors and shall also be specified in the building’s lease agreement. - Compliance with South Coast Air Quality Management District (AQMD) Rule 402 “Nuisance” is required. Rule 402 states that air contaminants and other materials shall not be discharged from any source whatsoever in quantities that would cause injury, detriment, nuisance, or annoyance to a considerable number of persons or the public, or which endanger the comfort, repose, health, or safety of any such persons or the public, or which cause, or have a natural tendency to cause, injury or damage to business or property. Public nuisance violations can occur when a considerable number of individuals complain to AQMD of odors, paint overspray, or other bothersome conditions that appear to be related to the operation of a business in the neighboring vicinity. | All | Project Applicant; Project Construction Contractors | City of Moreno Valley Land Development Division, Planning Division, and Building and Safety Division | Prior to the issuance of grading permits and building permits. |  |  |  |  |
| Biological Resources |  |  |  |  |  |  |  |  |
| MM 4.4-1 Within 30 days prior to grading, a qualified biologist shall conduct a survey of the property and make a determination regarding the presence or absence of the burrowing owl in accordance with the Burrowing Owl Survey Instructions for the Western Riverside MSHCP Area. The determination shall be documented in a report and shall be submitted, reviewed, and accepted by the City of Moreno Valley Planning Division prior to the issuance of a grading permit and subject to the following provisions: - In the event that the pre-construction survey identifies that no burrowing owls are present on the property, a grading permit may be issued without restriction. In the event that the pre-construction survey identifies the presence of at least one individual but less than three (3) mating pairs of burrowing owl, then prior to the issuance of a grading permit and prior to the commencement of ground-disturbing activities on the property, the qualified biologist shall passively or actively relocate any burrowing owls. Passive relocation, including the required use of one-way doors to exclude owls from the site and the collapsing of burrows, will occur if the biologist determines that the proximity and availability of alternate habitat is suitable for successful passive relocation. Passive relocation shall follow CDFW relocation protocol and shall only occur between September 15 and February 1. If proximate alternate habitat is not present as determined by the biologist, active relocation shall follow CDFW relocation protocol. The biologist shall confirm in writing that the species has fledged the site or been relocated prior to the issuance of a grading permit. In the event that the pre-construction survey identifies the presence of three (3) or more mating pairs of burrowing owl, the requirements of MSHCP Species-Specific Conservation Objectives 5 for the burrowing owl shall be followed. Objective 5 states that if the site (including adjacent areas) supports three (3) or more pairs of burrowing owls and supports greater than 35 acres of suitable Habitat, at least 90 percent of the area with long-term conservation value and burrowing owl pairs will be conserved onsite until it is demonstrated that Objectives 1-4 have been met. A grading permit shall only be issued, either: - Upon approval and implementation of a property-specific Determination of Biological Equivalent or Superior Preservation (DBESP) report for the western burrowing owl by the CDFW; or - A determination by the biologist that the site is part of an area supporting less than 35 acres of suitable Habitat, and upon passive or active relocation of the species following accepted CDFW protocols. Passive relocation, including the required use of one-way doors to exclude owls from the site and the collapsing or burrows, will occur if the biologist determines that the proximity and availability of alternate habitat is suitable for successful passive relocation. Passive relocation shall follow CDFW relocation protocol and shall only occur between September 15 and February 1. If proximate alternate Habitat is not present as determined by the biologist, active relocation shall follow CDFW protocol. The biologist shall confirm in writing that the species has fledged the site or been relocated prior to issuance of a grading permit. | All | Project Applicant; Project Biologist | City of Moreno Valley Planning Division & Land Development Division | Within 30 days prior to grading activities. |  |  |  |  |
| MM 4.4-2 As a condition of approval for all grading permits, vegetation clearing shall be prohibited during the migratory bird nesting season (February 1 through September 15), unless a migratory bird nesting survey is completed in accordance with the following requirements: - A migratory nesting bird survey of all vegetation that may support nesting birds shall be conducted by a qualified biologist within three (3) days prior to initiating vegetation clearing. | All | Project Applicant; Project Biologist | City of Moreno Valley Planning Division | - Within 3 days prior to initiating vegetation clearing. |  |  |  |  |
| MM 4.4-3 A copy of the migratory nesting bird survey results report shall be provided to the City of Moreno Valley Planning Division. If the survey identifies the presence of active nests, then the qualified biologist shall provide the City of Moreno Valley Planning Division with a copy of maps showing the location of all nests and an appropriate buffer zone around each nest sufficient to protect the nest from direct and indirect impacts. The size and location of all buffer zones, if required, shall be subject to review and approval by the City of Moreno Valley Planning Division and shall be no less than a 300-foot radius around the nest for non-raptors and a 500-foot radius around the nest for raptors. The nests and buffer zones shall be field checked weekly by a qualified biological monitor. The approved buffer zone shall be marked in the field with construction fencing, within which no vegetation clearing or ground disturbance shall commence until the qualified biologist and City Planning Division verify that the nests are no longer occupied and the juvenile birds can survive independently from the nests | All | Project Applicant; Project Biologist | City of Moreno Valley Planning Division | Within 3 days prior to initiating vegetation clearing. |  |  |  |  |
| Cultural Resources |  |  |  |  |  |  |  |  |
| MM 4.5-1 Prior to the issuance of a grading permit, the Project Applicant shall provide evidence to the City of Moreno Valley that a professional archaeologist has been retained by the Applicant to conduct monitoring of all mass grading and trenching activities. The Project Archaeologist shall have the authority to temporarily redirect earthmoving activities in the event that suspected archaeological resources are unearthed during Project construction. The Project Archaeologist, in consultation with the Monitoring Tribe(s), the Applicant and the City, shall develop a Cultural Resources Monitoring Plan (CRMP) to address the details, timing and responsibility of all archaeological and cultural resources activities that will occur on the Project site. Details in the CRMP shall include: a) Schedule for Project grading and development activities; b) The Project archeologist and the Monitoring Tribes(s) shall attend the pre-grading meeting with the City, the construction manager and any contractors and will conduct a mandatory Cultural Resources Worker Sensitivity Training to those in attendance. The Training will include a brief review of the cultural sensitivity of the Project and the surrounding area; what resources could potentially be identified during earthmoving activities; the requirements of the monitoring program; the protocols that apply in the event inadvertent discoveries of cultural resources are identified, including who to contact and appropriate avoidance measures until the find(s) can be properly evaluated; and any other appropriate protocols. All new construction personnel that will conduct earthwork or grading activities that begin work on the Project following the initial Training must take the Cultural Sensitivity Training prior to beginning work and the Project archaeologist and Monitoring Tribe(s) shall make themselves available to provide the training on an as-needed basis. c) The coordination of a monitoring schedule as agreed upon by the Monitoring Tribe(s), the Project archaeologist, and the Applicant; and d) The protocols and stipulations that the Applicant, City, Monitoring Tribe(s) and Project archaeologist will follow in the event of inadvertent cultural resources discoveries, including any newly discovered cultural resource deposits that shall be subject to a cultural resources evaluation. | All | Project Applicant | City of Moreno Valley Planning Division | Prior to the issuance of a grading permit. |  |  |  |  |
| MM 4.5-2 Prior to the issuance of a grading permit, the Applicant shall provide evidence to the City of Moreno Valley that appropriate Pechanga Band of Luiseño Indians and Soboba Band of Luiseño Indians tribal representatives (hereafter referred to as “Native American Tribal Representatives”) received a minimum of 30 days’ advance notice of all mass grading and trenching activities, and any monitoring agreements between the applicant and the Tribes as requested through the SB 18 process. Native American Tribal Representatives shall provide a copy of the signed agreement(s) prior to the issuance of a grading permit and the Tribal Representatives shall be notified of and allowed to attend the pre-grading meeting with the City and Project construction contractors and/or monitor all Project mass grading and trenching activities. The Native American Tribal Representatives shall have the authority to temporarily halt and redirect earth moving activities in the affected area in the event that suspected archaeological resources are unearthed. If the Native American Tribal Representatives suspect that an archaeological resource may have been unearthed, the Project Archaeologist or the Tribal Representatives shall immediately redirect grading operations in a 100-foot radius around the find to allow identification and evaluation of the suspected resource. In consultation with the Native American Tribal Representatives, the Project Archaeologist shall evaluate the suspected resource and make a determination of significance pursuant to California Public Resources Code Section 21083.2. If the resource is significant, Mitigation Measures MM 4.5.3 and MM 4.5.4 shall apply. | All | Project Applicant | City of Moreno Valley Planning Division | Prior to the issuance of a grading permit. |  |  |  |  |
| MM 4.5-3 A treatment plan shall be prepared by the Project Archaeologist and expeditiously reviewed by the interested Native American Tribal Representatives and the City Planning Division and implemented by the Project Archaeologist to protect the identified archaeological resource(s) from damage and destruction. If a significant archaeological resource(s) is discovered on the property, ground disturbing activities shall be temporarily suspended 100 feet around the resource(s) until the treatment plan is implemented. The Project Archaeologist, interested Native American Tribal Representatives, and the City Planning Division shall confer regarding mitigation of the discovered resource(s). | All | Project Applicant / Landowner; Project Construction Contractor; Project Archaeologist | City of Moreno Valley Planning Division | During grading operations |  |  |  |  |
| MM 4.5-4 In the event that Native American cultural resources are discovered during the course of grading, the following procedures shall be carried out for treatment and final disposition of the discoveries: a) The landowner(s) shall relinquish ownership of all cultural resources, including sacred items, burial goods, and all archaeological artifacts and non-human remains as part of the required mitigation for impacts to cultural resources. The artifacts shall be relinquished through one or more of the following methods and evidence of such shall be provided to the City of Moreno Valley Planning Department: i. Accommodate the process for Preservation-In-Place / Onsite reburial of the discovered items with the consulting Native American tribes or bands, as detailed in the treatment plan prepared by the Project Archaeologist under Mitigation Measure MM 4.5-3. This shall include measures and provisions to protect the future reburial area from any future impacts. Reburial shall not occur until all cataloguing and basic recordation have been completed; ii. A curation agreement with an appropriate qualified repository within Riverside County that meets federal standards per 36 CFR Part 79; therefore, the resources would be professionally curated and made available to other archaeologists / researchers for further study. The collections and associated records shall be transferred, including title, to an appropriate curation facility within Riverside County, to be accompanied by payment of the fees necessary for permanent curation; iii. For purposes of conflict resolution, if more than one Native American tribe or band is involved with the project and cannot come to an agreement as to the disposition of cultural materials, they shall be curated at the Western Science Center by default. | All | Landowner; Project Archaeologist | City of Moreno Valley Planning Division | In the event that Native American cultural resources are discovered during grading operations |  |  |  |  |
| MM 4.5-5 Prior to grading permit issuance, the City shall verify that the following note is included on the Grading Plan: “If any suspected archaeological resources are discovered during ground-disturbing activities and the Project Archaeologist or Native American Tribal Representatives are not present, the construction supervisor is obligated to halt work in a 100-foot radius around the find and call the Project Archaeologist and the Tribal Representatives to the site to assess the significance of the find." | All | Project Applicant | City of Moreno Valley Planning Division | Prior to grading permit issuance. |  |  |  |  |
| MM 4.5-6 Prior to the issuance of a grading permit, the Project Applicant shall provide evidence to the City of Moreno Valley that a qualified paleontologist has been retained by the Project Applicant to conduct monitoring of excavation activities and has the authority to halt and redirect earthmoving activities in the event that suspected paleontological resources are unearthed. | All | Project Applicant; Project Paleontologist | City of Moreno Valley Planning Division | Prior to issuance of grading permit |  |  |  |  |
| MM 4.5-7 The paleontological monitor shall conduct full-time monitoring during grading and excavation operations in undisturbed, very old alluvial fan sediments and shall be equipped to salvage fossils if they are unearthed to avoid construction delays and to remove samples of sediments that are likely to contain the remains of small fossil invertebrates and vertebrates. The paleontological monitor shall be empowered to temporarily halt or divert equipment to allow of removal of abundant and large specimens in a timely manner. Monitoring may be reduced if the potentially fossiliferous units are not present in the subsurface, or if present, are determined upon exposure and examination by qualified paleontological personnel to have a low potential to contain or yield fossil resources. | - All | - Project Paleontologist | City of Moreno Valley Planning Division | On-going during construction |  |  |  |  |
| MM 4.5-8 Recovered specimens shall be properly prepared to a point of identification and permanent preservation, including screen washing sediments to recover small invertebrates and vertebrates, if necessary. Identification and curation of specimens into a professional, accredited public museum repository with a commitment to archival conservation and permanent retrievable storage, such as the Western Science Museum in Hemet, California, is required for significant discoveries. | - All | - Project Paleontologist | City of Moreno Valley Planning Division | Prior to grading permit final inspection. |  |  |  |  |
| MM 4.5-9 A final monitoring and mitigation report of findings and significance shall be prepared, including lists of all fossils recovered, if any, and necessary maps and graphics to accurately record the original location of the specimens. The report shall be submitted to the City of Moreno Valley prior to building final. | - All | - Project Paleontologist | City of Moreno Valley Planning Division | Prior to building final. |  |  |  |  |
| Greenhouse Gas Emissions |  |  |  |  |  |  |  |  |
| MM 4.6-1 Prior to issuance of a building permit, the City of Moreno Valley shall verify that the roofs for Buildings #1, #2, #3, and #4 are designed to support solar panels. The entire roof area of each building is not required to support panels; the portion of the roof that is to support panels shall be determined by the City and the building’s architect at time of building design and building permit issuance. | All | Project Applicant | City of Moreno Valley Planning Division & Building and Safety Division | Prior to the issuance of building permits. |  |  |  |  |
| MM 4.6-2 Prior to building final, the City of Moreno Valley shall verify that the parking lot is marked in compliance with the California Green Building Standards Code, which requires that a certain number of parking spaces be designated for any combination of low-emitting, fuel-efficient and carpool/vanpool vehicles. The designated parking stalls are required to be painted “Clean Air Vehicle.” | All | Project Applicant | City of Moreno Valley Planning Division | Prior to building final. |  |  |  |  |
| MM 4.6-3 Prior to issuance of building permits for the landscape plan, the City of Moreno Valley shall review landscape plans to verify that trees will be planted in locations where tree placement would assist with passive solar heating and cooling of the structure, while also avoiding interference with vehicle movements and building operations. | All | Project Applicant | City of Moreno Valley Planning Division | Prior to building final. |  |  |  |  |
| MM 4.6-4 Prior to the approval of permits and approvals that would permit cold storage in Buildings #1, #2, #3, and/or #4, the Project Applicant shall provide information to the City of Moreno Valley demonstrating that the cooling system design is energy efficient. | All | Project Applicant | City of Moreno Valley Planning Division & Building and Safety Division | Prior to the approval of permits and approvals that would permit cold storage in Buildings #1, #2, #3, and/or #4. |  |  |  |  |
| Hazards and Hazardous Materials |  |  |  |  |  |  |  |  |
| MM 4.7-1 Prior to the issuance of building permits, a photometric plan shall be submitted to the City of Moreno Valley and approved. Any outdoor lighting installed shall be hooded or shielded to prevent either the spillage of lumens or reflection into the sky. Outdoor lighting shall be downward facing. | All | Project Applicant | City of Moreno Valley Planning Division & Building and Safety Division | Prior to issuance of building permits. |  |  |  |  |
| MM 4.7-2 The following uses shall be prohibited: - Any use which would direct a steady light or flashing light of red, white, green, or amber colors associated with airport operations toward an aircraft engaged in an initial straight climb following takeoff or toward an aircraft engaged in a straight final approach toward a landing at an airport, other than an FAA-approved navigational signal light or visual approach slope indicator. Any use which would cause sunlight to be reflected towards an aircraft engaged in an initial straight climb following takeoff or towards an aircraft engaged in a straight final approach towards a landing at an airport. Any use which would generate smoke or water vapor or which would attract large concentrations of birds, or which may otherwise affect safe air navigation within the area. (Such uses include landscaping utilizing water features, aquaculture, production of cereal grains, sunflower, and row crops, composting operations, trash transfer stations that are open on one or more sides, recycling centers containing putrescible wastes, construction and demolition debris facilities, fly ash disposal, and incinerators.) Any use which would generate electrical interference that may be detrimental to the operation of aircraft and/or aircraft instrumentation. In Buildings 3 and 4: Children's schools, day care centers, libraries, hospitals, skilled nursing and care facilities, congregate care facilities, noise sensitive outdoor nonresidential uses and hazards to flight. | All | Project Applicant | City of Moreno Valley Planning Division & Building and Safety Division | Prior to the issuance of building permit; prior to building final. |  |  |  |  |
| MM 4.7-3 The “Notice of Airport in Vicinity,” included in the ALUC’s October 8, 2015 staff report, shall be given to all prospective purchasers of the property and tenants of the buildings, and shall be recorded as a deed notice. Prior to building final, the Project Applicant shall provide to the City of Moreno Valley a copy of the title report and a model lease agreement for the subject property that includes the airport proximity notice. |  | Project Applicant | City of Moreno Valley Planning Division & Building and Safety Division | Prior to building final. |  |  |  |  |
| MM 4.7-4 The proposed detention basins on the site (including water quality management basins) shall be designed so as to provide for a maximum 48-hour detention period following the conclusion of the storm event for the design storm (may be less, but not more), and to remain totally dry between rainfalls. Vegetation in and around the detention basins that would provide food or cover for bird species that would be incompatible with airport operations shall not be utilized in project landscaping. Trees shall be spaced so as to prevent large expanses of contiguous canopy, when mature. Landscaping in and around the detention basins located westerly of the Perris Valley Storm Drain Channel shall not include trees that produce seeds, fruits, or berries. | All | Project Applicant | City of Moreno Valley Building and Safety Division, Land Development Division, and Planning Division | Prior to the issuance of grading permits; prior to the issuance of building permits. |  |  |  |  |
| MM 4.7-5 March Air Reserve Base must be notified of any land use having an electromagnetic radiation component to assess whether a potential conflict with Air Base radio communications could result. Sources of electromagnetic radiation include radio wave transmission in conjunction with remote equipment inclusive of irrigation controllers, access gates, etc. All sources of electromagnetic radiation shall be noted on building plans and tenant improvement plans. | All | Project Applicant | City of Moreno Valley Planning Division & Building and Safety Division | Prior to issuance of building permits. |  |  |  |  |
| MM 4.7-6 The Federal Aviation Administration has conducted aeronautical studies of each of the proposed buildings (Aeronautical Study Nos. 2015-AWP-8676-0E through 2015-AWP-8679-0E) and has determined that neither marking nor lighting of these structures is necessary for aviation safety. However, if marking and/or lighting for aviation safety are accomplished on a voluntary basis, such marking and/or lighting (if any) shall be installed in accordance with Federal Advisory Circular 70/7460-1 K Change 2 and shall be maintained therewith for the life of the Project. All voluntary marking and/or lighting shall be identified on building plans. | All | Project Applicant | City of Moreno Valley Planning Division & Building and Safety Division | Prior to issuance of building permits. |  |  |  |  |
| MM 4.7-7 The maximum height of Building 1 shall not exceed 60 feet above ground level, and the maximum elevation at top point (including any roof-mounted equipment) shall not exceed 1,549 feet above mean sea level. | Building 1 | Project Applicant | City of Moreno Valley Planning Division & Building and Safety Division | Prior to the issuance of building permits. |  |  |  |  |
| MM 4.7-8 The maximum height of Building 2 shall not exceed 52 feet above ground level, and the maximum elevation at top point (including any roof-mounted equipment) shall not exceed 1,541 feet above mean sea level. | Building 2 | Project Applicant | City of Moreno Valley Planning Division & Building and Safety Division | Prior to issuance of building permits. |  |  |  |  |
| MM 4.7-9 The maximum height of Building 3 shall not exceed 52 feet above ground level, and the maximum elevation at top point (including any roof-mounted equipment) shall not exceed 1,532 feet above mean sea level. | Building 3 | Project Applicant | City of Moreno Valley Planning Division & Building and Safety Division | Prior to issuance of building permits. |  |  |  |  |
| MM 4.7-10 The maximum height of Building 4 shall not exceed 52 feet above ground level, and the maximum elevation at top point (including any roof-mounted equipment) shall not exceed 1,545 feet above mean sea level. | Building 4 | Project Applicant | City of Moreno Valley Planning Division & Building and Safety Division | Prior to issuance of building permits. |  |  |  |  |
| MM 4.7-11 The specific coordinates, heights, and top point elevations of the proposed buildings shall not be amended without further review by the Airport Land Use Commission and the Federal Aviation Administration; provided, however, that reduction in building height or elevation shall not require further review by the Airport Land Use Commission. | All | Project Applicant | City of Moreno Valley Planning Division | Prior to issuance of building permits. |  |  |  |  |
| MM 4.7-12a Temporary construction equipment used during actual construction of Building 1 shall not exceed a height of 60 feet, unless separate notice is provided to the Federal Aviation Administration through the Form 7460-1 process. | Building 1 | Project Applicant | City of Moreno Valley Planning Division & Building and Safety Division | Prior to issuance of building permits. |  |  |  |  |
| MM 4.7-12b Temporary construction equipment used during actual construction of Buildings 2, 3, and 4 shall not exceed a height of 52 feet, unless separate notice is provided to the Federal Aviation Administration through the Form 7460-1 process. | Buildings 2, 3, and 4 | Project Applicant | City of Moreno Valley Planning Division & Building and Safety Division | Prior to issuance of building permits. |  |  |  |  |
| MM 4.7-13 Within five (5) days after construction of each of the buildings reaches its greatest height and prior to building final, FAA Form 7460-2 (Part II), Notice of Actual Construction or Alteration, shall be completed by the project proponent or his/her designee and e-filed with the Federal Aviation Administration, with documentation provided to the City of Moreno Valley. (Instructions are available at https://oeaaa.faa.gov.) This requirement is also applicable in the event the project is abandoned or a decision is made not to construct the applicable building. | All | Project Applicant | City of Moreno Valley Planning Division & Building and Safety Division | Within five (5) days after construction of each building and prior to building final. |  |  |  |  |
| Hydrology and Water Quality |  |  |  |  |  |  |  |  |
| - MM 4.8-1 Prior to building final, the Project Applicant shall provide evidence to the City of Moreno Valley that an application for a Final Letter of Map Revision (LOMR) has been submitted to FEMA to permanently remove the development area from the FEMA 100-year floodplain, and shall demonstrate to the satisfaction of the City of Moreno Valley that the finished floor height of the structure is outside the 100-year floodplain elevation as mapped by FEMA. | Buildings 1 and 2 | Project Applicant | City of Moreno Valley Building and Safety Division & Land Development Division | Prior to building final |  |  |  |  |
| Noise |  |  |  |  |  |  |  |  |
| MM 4.10-1 All construction activities shall comply with the City of Moreno Valley Noise Ordinance (Chapter 11.80 of the City of Moreno Valley Municipal Code). This requirement shall be noted on all grading and building plans and in bid documents issued to construction contractors. | All | Project Applicant; Project Construction Contractor | City of Moreno Valley Planning Division, Building and Safety Division, and Land Development Division | Prior to issuance of grading and building permits. |  |  |  |  |
| MM 4.10-2 Prior to the issuance of grading permits and building permits that would authorize grading and paving construction activities within 280 feet of Indian Street between Superior Avenue and the Perris Valley Storm Drain Channel, the construction contractor shall install a minimum 6-foot high temporary noise control barrier at the southeast corner of Parcel 1 (the Building 1 site) extending northward approximately 400 feet along Indian Street. Alternatively, with the approval of the property owner at 16950 Indian Street (noise receiver location R8), the temporary noise barrier can instead be installed along the west property line of that existing residential home. The temporary noise control barrier must present a solid face from top to bottom and must be a minimum of 6 feet high. The temporary noise control barrier shall comply with the following: - The noise barrier shall be constructed using an acoustical blanket (e.g. vinyl acoustic curtains or quilted blankets) attached to the construction site perimeter fence or equivalent temporary fence posts. The noise barrier shall be maintained in good repair during the duration of grading and paving activities on Parcel 1. Any damage shall be promptly repaired. Gaps, holes, or weaknesses in the barrier or openings between the barrier and the ground shall be promptly repaired. The noise control barrier and associated elements shall be completely removed upon the conclusion of the grading and paving construction activity on Parcel 1. In the event that the noise barrier is constructed at 16950 Indian Street (noise receiver location R8), documentation of property owner approval to construct the noise barrier shall be provided to the City of Moreno Valley Planning Division prior to construction of the barrier. | Building 1 | Project Applicant; Project Construction Contractor | City of Moreno Valley Planning Division, Building and Safety Division, and Land Development Division | Prior to the issuance of grading permits and building permits that would authorize grading and paving construction activities within 280 feet of Indian Street between Superior Avenue and the Perris Valley Storm Drain Channel. |  |  |  |  |
| MM 4.10-3 Prior to issuance of any grading and building permits, the City of Moreno Valley shall review grading and building plans to ensure the following notes are included on the plans. Project contractors shall be required to comply with these notes and maintain written records of such compliance that can be inspected by the City of Moreno Valley upon request. - Construction contractors shall equip all construction equipment, fixed or mobile, with properly operating and maintained mufflers, consistent with the manufacturer’s standards. The construction contractor shall place all stationary equipment so that emitted noise is directed away from noise sensitive receivers located east and northeast of the Project site. During construction activities on Parcel 1, construction contractors shall locate equipment staging in the vicinity of the intersection of Cosmos Street and Krameria Avenue to create distance between construction-related noise sources and noise-sensitive receivers located east and northeast of Indian Street. Haul truck deliveries shall use approved truck routes and occur during the same hours specified for construction equipment (7:00 a.m. to 8:00 p.m. on any given day) by the Moreno Valley Municipal Code Section 11.80.030.D.7 The construction contractor shall prepare a haul route exhibit for review and approval by the City of Moreno Valley Public Works Department, Land Development Division and shall design delivery routes to minimize exposure of sensitive land uses or residential dwellings to haul truck-related noise (Moreno Valley Municipal Code Section 8.21.050.H.7). | All, except sub-part b) shall only apply to Building 1 | Project Applicant; Project Contractors. | City of Moreno Valley Planning Division, Building and Safety Division, and Land Development Division | Prior to issuance of any grading and building permits. |  |  |  |  |
| MM 4.10-4 Prior to the issuance of building permits, the City of Moreno Valley shall review building plans to ensure that the following notes are included on the plans. Contractors shall be required to comply with these notes and maintain written records of such compliance that can be inspected by the City of Moreno Valley upon request. Additionally, prior to building permit issuance, the Project’s property owner(s) shall provide documentation to the City of Moreno Valley verifying that provisions are made in the buildings’ lease agreements that inform tenants of the following: - All on-site operating equipment under the control of the building user that is used in outdoor areas (including but not limited to trucks, tractors, forklifts, and hostlers), shall be operated with properly functioning and well-maintained mufflers. - Speed bumps are not allowed. Quality pavement conditions shall be maintained on the property that is free of vertical deflection (i.e. speed bumps) to minimize truck noise. | All | Project Applicant; Project Contractors; Project’s Property Owner(s). | City of Moreno Valley Planning Division & Building and Safety Division | Prior to the issuance of building permits. |  |  |  |  |
| Transportation and Traffic |  |  |  |  |  |  |  |  |
| MM 4.11-1 Prior to the issuance of the first grading permit, the traffic signal at the Heacock Street / San Michele Road intersection shall be modified to provide overlap phasing on the westbound right turn lane. | Building 1, 2, 3, or 4, whichever is subject to the first grading permit. | Project Applicant | City of Moreno Valley Planning Division Land Development Division, and Transportation Engineering Division | Prior to the issuance of the first grading permit. |  |  |  |  |
| MM 4.11-2 Prior to the issuance of grading or building permits, the Project Applicant shall prepare and submit a temporary traffic control plan to the City of Moreno Valley for approval. The temporary traffic control plan shall comply with the applicable requirements of the California Manual on Uniform Traffic Control Devices. A requirement to comply with the temporary traffic control plan shall be noted on all grading and building plans and also shall be specified in bid documents issued to prospective construction contractors. The temporary traffic control plan shall require the following: - The construction contractor shall assure that construction-related trips, including employee trips and delivery trucks, shall utilize the most direct route between the Project site and the I-215 freeway via Harley Knox Boulevard. | All | Project Applicant; Construction Contractor | City of Moreno Valley Planning Division, Building and Safety Division, and Transportation Engineering Division | Prior to the issuance of grading or building permits and bid documents. |  |  |  |  |
| MM 4.11-3 Prior to building final for the Project’s first building, the Project Applicant shall assure the Heacock Street / Cactus Avenue intersection is improved with the following geometrics: - Re-stripe the two northbound left turn lanes to provide 315 feet of lane storage for each lane. | Building 1, 2, 3, or 4, whichever is subject to the first building final. | Project Applicant | City of Moreno Valley Planning Division, Building and Safety Division, and Transportation Engineering Division | Prior to building final for the first building. |  |  |  |  |
| MM 4.11-4 Prior to building final for the Project’s first building, a traffic signal (as programmed under the City of Moreno Valley Development Impact Fee program) shall be installed at the Heacock Street / Gentian Avenue intersection. | Building 1, 2, 3, or 4, whichever is subject to the first building final. | Project Applicant | City of Moreno Valley Planning Division, Building and Safety Division, and Transportation Engineering Division | Prior to building final for the first building. |  |  |  |  |
| MM 4.11-5 Prior to building final for the Project’s first building, a traffic signal (as programmed under the City of Moreno Valley Development Impact Fee program) shall be installed at the Heacock Street / Iris Avenue intersection. | Building 1, 2, 3, or 4, whichever is subject to the first building final. | Project Applicant | City of Moreno Valley Planning Division, Building and Safety Division, and Transportation Engineering Division | Prior to building final for the first building. |  |  |  |  |
| MM 4.11-6 In the event a bridge has been constructed over the Perris Valley Storm Drain Channel to connect Indian Street on the north/south sides of the Channel prior to building final for the Project’s first building, then the Project Applicant shall use reasonable efforts to make a fee payment to the City of Perris that shall be used to modify the traffic signal at the Indian Street / Harley Knox Boulevard intersection to provide overlap phasing on the southbound right turn lane. | Building 1, 2, 3, or 4, whichever is subject to the first building final. | Project Applicant | City of Moreno Valley Planning Division, Building and Safety Division, and Transportation Engineering Division | In the event a bridge has been constructed over the Perris Valley Storm Drain Channel to connect Indian Street on the north/south sides of the Channel prior to the issuance of the Project’s first building final. |  |  |  |  |
| MM 4.11-7 Prior to issuance of building permits, the Project shall comply with the City of Moreno Valley Development Impact Fee (DIF) program, which requires the payment of a fee to the City (less fee credits), a portion of which is applied to reduce traffic congestion by funding the installation of roadway improvements. | All | Project Applicant | City of Moreno Valley Planning Division, Building and Safety Division, Transportation Engineering Division, and Land Development Division | Prior to the issuance of building permits. |  |  |  |  |
| MM 4.11-8 Prior to issuance of building permits, the Project shall comply with the Transportation Uniform Mitigation Fee (TUMF) program, which funds off-site regional transportation improvements. | All | Project Applicant | City of Moreno Valley Planning Division, Building and Safety Division, Transportation Engineering Division, and Land Development Division | Prior to the issuance of building permits. |  |  |  |  |
| MM 4.11-9 Prior to issuance of building final for Buildings 1, 2, 3, and 4 the Project Applicant shall make a fair share fee payment to the City of Moreno Valley for the roadway improvements listed in Table 6-6 and Table 7-6 of the “Moreno Valley Logistics Center Traffic Impact Analysis,” prepared by Urban Crossroads (dated February 26, 2016), that are located within the geographical limits of the City of Moreno Valley. These roadway improvements are not included within the City of Moreno Valley’s Development Impact Fee (DIF) program. The fair share fee attributable to Buildings 1, 2, 3, and 4 shall be calculated according to the percentages specified in EIR Table 4.11-35, Project Fair Share Calculations. | All | Project Applicant | City of Moreno Valley Planning Division, Building and Safety Division, and Transportation Engineering Division | Prior to issuance of building final for Buildings 1, 2, 3, and/or 4. |  |  |  |  |
| MM 4.11-10 Prior to issuance of the building final for Buildings 1, 2, 3, 4 and 4, the Project Applicant shall use reasonable efforts to make a fair share fee payment to the March Joint Powers Authority, for the roadway improvements listed in Table 6-6 and Table 7-6 of the “Moreno Valley Logistics Center Traffic Impact Analysis,” prepared by Urban Crossroads (dated February 26, 2016), that are located within the March Joint Powers Authority’s jurisdiction. The needed roadway improvements are not included within an existing mitigation program where the Project can participate. The fair share fee attributable to Buildings 1, 2, 3, and 4 shall be calculated according to the percentages specified in EIR Table 4.11-35, Project Fair Share Calculations. | All | Project Applicant | City of Moreno Valley Planning Division, Building and Safety Division, and Transportation Engineering Division | Prior to the issuance of building final for Buildings 1, 2, 3, and/or 4. |  |  |  |  |
| MM 4.11-11 Prior to issuance of the building final for Buildings 1, 2, 3, and 4, the Project Applicant shall use reasonable efforts to make a fair share fee payment to the City of Perris, for the improvements listed in Table 6-6 and Table 7-6 of the “Moreno Valley Logistics Center Traffic Impact Analysis,” prepared by Urban Crossroads (dated November 18, 2015), that are located within the City of Perris’ jurisdiction. The needed roadway improvements are not included within an existing mitigation program where the Project can participate. The fair share fee attributable to Buildings 1, 2, 3, and 4 shall be calculated according to the percentages specified in EIR Table 4.11-35, Project Fair Share Calculations. | All | Project Applicant | City of Moreno Valley Planning Division, Building and Safety Division, and Transportation Engineering Division | Prior to issuance of the building final for Buildings 1, 2, 3, and/or 4. |  |  |  |  |
| MM 4.11-12 In the event that Caltrans prepares a valid study, as defined below, that identifies fair share contribution funding sources attributable to and paid from private and public development to supplement other regional and State funding sources necessary undertake improvements to I-215 and SR-91 in the Project study area, then the Project Applicant shall use reasonable efforts to pay the applicable fair share amount to Caltrans. The study shall include fair share contributions related to private and or public development based on nexus requirements contained in the Mitigation Fee Act (Govt. Code § 66000 et seq.) and 14 Cal. Code of Regs. § 15126.4(a)(4) and, to this end, the study shall recognize that impacts to Caltrans I-215 and SR-91 facilities that are not attributable to development located within the City of Moreno Valley are not required to pay in excess of such developments’ fair share obligations. The fee study shall also be compliant with Government Code § 66001(g) and any other applicable provisions of law. The study shall set forth a timeline and other relevant criteria for implementation of the recommendations contained within the study to the extent the other agencies agree to participate in the fee study program. In the event the study has been prepared, the Project Applicant shall use reasonable efforts to pay the fair share amount to Caltrans. If Caltrans chooses to accept the Project Applicant’s fair share payment, Caltrans shall apply the payment to the fee program adopted by Caltrans or agreed upon by the Project Applicant and Caltrans as a result of the fair share fee study. Caltrans shall only accept the fair share payment if the fair share fee study has been completed. If, within five years from the date that the first building permit is issued for the Project, Caltrans has not completed the fair share fee study, then the Project Applicant shall have no further obligation to comply with this mitigation measure. | All | Project Applicant | City of Moreno Valley Planning Division, Building and Safety Division, and Transportation Engineering Division | In the event that Caltrans prepares a valid study that identifies fair share contribution funding sources in the Project study area. |  |  |  |  |